Edge Computing for Autonomous Driving: Opportunities and Challenges

This article surveys the designs of autonomous driving edge computing systems. In addition, it presents the security issues in autonomous driving as well as how edge computing designs can address these issues.

By SHAOSHAN LIU , Senior Member IEEE, LIANGKAI LIU, JIE  TANG, Member IEEE, BO YU, Member IEEE, YIFAN WANG, AND WEISONG SHI, Fellow IEEE

ABSTRACT | Safety is the most important requirement for autonomous vehicles; hence, the ultimate challenge of design- ing an edge computing ecosystem for autonomous vehicles is to deliver enough computing power, redundancy, and secu- rity so as to guarantee the safety of autonomous vehicles. Specifically, autonomous driving systems are extremely com- plex; they tightly integrate many technologies, including sens- ing, localization, perception, decision making, as well as the smooth interactions with cloud platforms for high-definition (HD) map generation and data storage. These complexities impose numerous challenges for the design of autonomous driving edge computing systems. First, edge computing sys- tems for autonomous driving need to process an enormous amount of data in real time, and often the incoming data from different sensors are highly heterogeneous. Since autonomous driving edge computing systems are mobile, they often have very strict energy consumption restrictions. Thus, it is imper- ative to deliver sufficient computing power with reasonable energy consumption, to guarantee the safety of autonomous vehicles, even at high speed. Second, in addition to the edge system design, vehicle-to-everything (V2X) provides redun- dancy for autonomous driving workloads and alleviates strin-

Manuscript received February 10, 2019; revised April 6, 2019; accepted

May 5, 2019. Date of publication June 24, 2019; date of current version August 5, 2019. (Corresponding author: Jie Tang.)

S. Liu and B. Yu are with PerceptIn, Fremont, CA 94539 USA.

L. Liu and W. Shi are with the Department of Computer Science, Wayne State University, Detroit, MI 48202 USA.

J. Tang is with the School of Computer Science & Engineering, South China University of Technology, Guangzhou 510330, China (e-mail: cstangjie@scut.edu.cn).

Y. Wang is with Wayne State University, Detroit, MI 48202 USA, and also with the Institute of Computing Technology, Chinese Academy of Sciences, Beijing 100190, China.

Digital Object Identifier 10.1109/JPROC.2019.2915983

gent performance and energy constraints on the edge side. With V2X, more research is required to define how vehi-  cles cooperate with each other and the infrastructure. Last, safety cannot be guaranteed when security is compromised. Thus, protecting autonomous driving edge computing systems against attacks at different layers of the sensing and comput- ing stack is of paramount concern. In this paper, we review state-of-the-art approaches in these areas as well as explore potential solutions to address these challenges.

KEYWORDS | Connected and autonomous vehicles (CAVs); edge computing; heterogeneous computing; security; vehicle- to-everything (V2X); vehicular operating system.

## INT R OD UCTION

The design goal of autonomous driving edge computing systems is to guarantee the safety of autonomous vehicles. This is extremely challenging, as autonomous vehicles need to process an enormous amount of  data  in  real  time (as high as 2 GB/s) with extremely tight latency constraints [1]. For instance, if an autonomous vehicle travels at 60 miles per hour (mph), and thus about 30  m  of braking distance, this requires the autonomous driving system to predict potential dangers up to a few seconds before they occur. Therefore, the faster the autonomous driving edge computing system performs these complex computations, the safer the autonomous vehicle is.

Specifically, autonomous driving systems are extremely complex; they tightly integrate many technologies, includ- ing  sensing,  localization,  perception, decision  making, as well as the smooth interaction with cloud platforms for high-definition (HD) map generation and data storage [2].

These complexities pose many challenges for the design of  autonomous  driving  edge  computing  systems, just to

0018-9219 © 2019 IEEE. Personal use is permitted, but republication/redistribution requires IEEE permission.

See http://www.ieee.org/publications_standards/publications/rights/index.html for more information.

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1697

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

name a few. First, they need to enable the interactions between the functional modules with low overheads, and this requires a lightweight operating system. Second, they need to process an enormous amount of data in real time, and often the incoming data from different sensors are highly heterogeneous. In addition, since they are mobile systems, they often have very strict energy consumption restrictions. This requires a high-performance and energy- efficient hardware system.

In addition to the edge system design, vehicle-to- everything (V2X) technologies should  also get  involved in the communication of the edge system. V2X provides redundancy for autonomous driving workloads; it also alleviates stress on the edge computing system. We believe this is a promising  approach,  but the key is to identify     a sweet spot between the tradeoffs  of  fully  relying  on the edge computing system versus fully relying on the V2X infrastructure. Hence, exploring how V2X enables autonomous vehicles to cooperate with each other and with the infrastructure remains an open research topic.

Having a high-performance and energy-efficient edge computing system is not enough, as attackers may turn an autonomous vehicle into a dangerous weapon by hacking into any layer of the autonomous driving sensing and computing stack. More research is required to identify all possible attack surfaces as well as protection mechanisms before autonomous vehicles are released on public roads.

To summarize, the overarching challenge of autonomous  driving  edge  computing  systems  design  is to efficiently integrate the functional modules, including interactions with edge servers  and  V2X  infrastructure,  to process a massive amount of heterogeneous data  in real time, within a limited energy budget, and without sacrificing  the  security  of  the  users.   In   this   paper,  we  review  state-of-the-art  approaches  in   these   areas as well as explore potential solutions to address these challenges.

Before delving into the details of the autonomous driving edge computing system designs, let us first define the autonomous driving computing ecosystem. As illus- trated in Fig. 1, each autonomous vehicle is equipped  with an edge computing system, which integrates all the real-time functional modules, such as localization, percep- tion, planning and control, and so on. Then, each vehicle communicates with edge servers, and eventually with the central cloud, through existing 3G/4G/5G communica- tion networks. In addition, the vehicles can communicate with the road side units (RSUs) through either the 5G networks or the dedicated short-range communications (DSRC) networks; this is a typical vehicle-to-infrastructure (V2I) scenario. Moreover, the vehicles can communicate with each other through the DSRC networks, and this is a typical vehicle-to-vehicle (V2V) scenario.

The remainder of this paper  is organized  as follows.  In Section II, we review the background of autonomous driving technologies and help readers understand the com- plexities of autonomous driving systems. In Section III,

Fig. 1. Autonomous driving edge computing architecture.

the latest progress on the designs of autonomous driving edge computing systems is presented and discussed. In Section IV, we focus on how V2X alleviates the stress on edge computing systems. In Section V, we review security issues in autonomous driving and how edge computing designs address these issues. We conclude in Section VI.

## AU T O N O M O U S D R I V I N G TE CH N O LO G I E S

As detailed in [3], autonomous driving is not one tech- nology but rather an integration of  many  technologies. As is shown in Fig. 2, the autonomous driving technol- ogy stack consists of three major subsystems: algorithms, including sensing, perception, and decision; a vehicular edge subsystem, including the operating system and hard- ware platform; and a cloud platform, including data stor- age, simulation, HD mapping, and deep learning model training.

# Algorithm Subsystem

The algorithm subsystem extracts meaningful informa- tion from sensor raw data to understand its environment and dynamically make decisions about its actions.

Fig. 2. Autonomous driving technology stack.

1698 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

  - Sensing: Normally, an autonomous vehicle consists of several major sensors. Since each type of sensor presents advantages and drawbacks, the data from multiple sensors must be combined. The sensor types include the following.

    - GNSS/IMU: The global navigation satellite system and inertial measurement unit (GNSS/IMU) system helps autonomous vehicles localize themselves by reporting both inertial updates and a global position estimate at a high rate, e.g., 200 Hz. While GNSS is a fairly accurate local- ization sensor at only 10 Hz, its update rate is too slow to provide real-time updates. Now, although an IMU’s accu- racy degrades with time and thus cannot be relied upon  to provide accurate position  updates over  long periods,  it  provides  updates more  frequently—at, or  higher than,

200 Hz. This should satisfy  the  real-time  requirement.  By combining GNSS and IMU, we provide accurate and real-time updates for vehicle localization.

    - LiDAR: Light detection and ranging (LiDAR) is used for mapping,  localization, and obstacle avoidance.  It works by bouncing a beam off surfaces and measuring the reflection time to determine distance. LiDAR is used to generate HD maps, localize a moving vehicle against HD maps, detect obstacles ahead, and so on.

    - Cameras: Cameras are mostly used for object recognition and object tracking tasks, such as lane detec- tion, traffic light detection, pedestrian detection, and more. To enhance autonomous vehicle safety, existing imple- mentations usually mount eight or more cameras around the car, and we use cameras to detect,  recognize,  and track objects in front, behind, and on both sides of the vehicle. These cameras usually run at 60 Hz and, when combined, generate around multiple gigabytes of raw data per second.

    - Radar and Sonar: The radar and sonar system is used for the last line of defense in  obstacle  avoidance. The data generated by radar and sonar show the distance from the nearest object in front of the vehicle’s path. When we detect that an object is not far ahead and that there may be a danger of a collision, the autonomous vehicle should apply the brakes or turn to avoid the obstacle. Therefore, the data generated by radar and sonar do not require much processing and are usually fed directly to the control processor—not through the main computation pipeline—to implement such urgent functions as swerving and applying the brakes.

  - Perception—Localization: The perception subsystem consumes incoming sensor data to understand the vehi- cle’s environment. The main tasks in autonomous driving perception include localization, object detection, object tracking, and so on.

While GNSS/IMU is used for localization, GNSS pro- vides fairly accurate localization results but with a slow update rate; IMU provides a fast update with less accu- rate results. We use Kalman filtering to combine the advantages of the two and provide accurate and real-time position updates [4]. Nonetheless, we cannot solely rely

on this combination for localization for three reasons:

1) the accuracy is only about one meter; 2) the GNSS signal has multipath problems, which means that the signal may bounce off buildings and introduce more noise; and 3) GNSS requires an unobstructed view of the sky so it fails to work in closed environments such as tunnels.

Cameras are used for localization. The pipeline line of Vision-based localization is simplified as follows: 1) by tri- angulating stereo image pairs, a disparity map is obtained and used to derive depth information for each point; 2) by matching salient features between successive stereo image frames in order to establish correlations between feature points in different frames, the motion between the past two frames is estimated; and 3) by comparing the salient features against those in the known map, we derive the current position of the vehicle [5].

LiDAR is used for localization, relying heavily on a parti- cle filter [6]. The point clouds generated by LiDAR provide a “shape description” of the environment, but it is hard   to differentiate individual points. By using a particle filter, the system compares a specific observed shape against the known map to reduce uncertainty. To localize a moving vehicle relative to these maps, we apply a particle filter method to correlate the LiDAR measurements with the map. The particle filter method has been demonstrated to achieve real-time localization with 10-cm accuracy and is effective in urban environments.

3)  Perception—Object Recognition and Tracking:  In recent years, we have seen the rapid development of deep learn- ing  technology,  which  achieves  significant  object  detec- tion and tracking accuracy. A convolution neural network (CNN)  is  a  type  of  deep  neural  network  (DNN)  that  is widely  used  in  object  recognition  tasks.  A  general  CNN evaluation pipeline usually consists of the following layers:

1) the convolution layer uses different filters to extract different features from the input image. Each filter con- tains a set of “learnable” parameters that will be derived after the training stage; 2) the activation layer decides whether to activate the target neuron; 3) the pooling layer reduces the spatial size of the representation to reduce the number of parameters and consequently the computation in the network; and last, 4) the fully connected layer connects all neurons to all activations in the previous  layer [7].

Once an object is identified using a CNN, next comes the automatic estimation of the trajectory of that object as it moves—or, object tracking. Object tracking technology  is used to track nearby moving vehicles, as well as people crossing the road, to ensure the current vehicle does not collide with moving objects. In recent years, deep learning techniques have demonstrated advantages in object track- ing compared to conventional computer vision techniques. By using auxiliary natural images, a stacked autoencoder is trained offline to learn generic image features that are more robust against variations in viewpoints and vehicle

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1699

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

positions. Then, the offline-trained model is applied for online tracking [8].

4) Decision: In the decision stage, action prediction,  path planning, and obstacle avoidance mechanisms are combined to generate an effective action plan in real time. One of the main challenges for human drivers when navi- gating through traffic is to cope with the possible actions of other drivers, which directly influences their own driving strategy. This is especially true when there are multiple lanes on the road or at a traffic change point.

To make sure that autonomous vehicles travel safely in these environments, the decision unit generates predic- tions of nearby vehicles and then decides on an action plan based on these predictions.  To  predict  the  actions of other  vehicles,  we  generate  a  stochastic  model  of the reachable position sets of the other traffic  partici- pants and associate these reachable sets with probability distributions [9].

Planning the path of an autonomous, responsive vehicle in a dynamic environment is a complex problem, especially when the vehicle is required to use its full maneuvering capabilities. One approach would be to use deterministic, complete algorithms to search all possible paths and utilize a cost function to identify the best path. However, this requires enormous computational resources and may be unable to deliver real-time navigation plans. To circum- vent this computational complexity and provide effective real-time path planning, probabilistic planners have been utilized [10].

Since safety is of paramount concern in autonomous driving, we should employ at least two  levels of  obsta- cle  avoidance  mechanisms  to  ensure   that   the   vehi- cle will not collide with obstacles. The first level is proactive and based on traffic predictions. The traffic prediction mechanism generates measures like time-to- collision or predicted-minimum-distance. Based on these measures, the obstacle avoidance mechanism is triggered to perform local path replanning. If the proactive mech- anism fails, the second-level reactive mechanism, using radar data, takes over. Once the radar detects an obstacle ahead of the path, it overrides the current controls to avoid the obstacle [11].

# Vehicular Edge Subsystem

The vehicular edge subsystem integrates the above- mentioned algorithms together to meet real-time and reliability requirements. There are three challenges to overcome: 1) the system needs to make sure that the processing pipeline is fast enough to consume the enor- mous amount of sensor data generated; 2)  if  a  part  of the system fails, it needs to be robust enough to recover from the failure; and 3) the system needs to perform all the computations under energy and resource constraints.

  - Hardware Architecture: First, we review  the  exist- ing computer architecture for autonomous driving tasks.

A more detailed review is found in [1]. The Nvidia PX platform is the current leading GPU-based solution for autonomous driving [12]. Each PX 2 consists of two Tegra systems-on-chip (SoCs) and two Pascal graphics proces- sors. Each GPU has its own dedicated memory, as well as specialized instructions for DNN acceleration. To deliver high throughput, each Tegra connects directly to the Pascal GPU using a PCI-E Gen 2 4 bus (total bandwidth:

×

4.0 GB/s). In addition, the dual CPU-GPU cluster is con- nected over the gigabit Ethernet, delivering 70 Gb/s. With optimized I/O architecture and DNN acceleration, each PX2 is able to perform 24 trillion deep-learning calcula- tions every second.

Texas Instruments’ TDA provides a DSP-based solution for autonomous driving. A TDA2x SoC consists of two floating-point C66x DSP cores and four fully program- mable vision accelerators, which are designed for vision processing functions. The vision accelerators provide eight- fold acceleration on vision tasks compared to an ARM Cortex-15 CPU while consuming less power [13]. Similarly, CEVA XM4 is another DSP-based autonomous driving com- puting solution. It is designed for computer vision tasks on video streams. The main benefit for using CEVA-XM4 is energy efficiency, which requires less than 30 mW for a 1080p video at 30 frames/s [14].

Altera’s Cyclone V SoC is one field-programmable gate array (FPGA)-based autonomous driving solution, which has been used in Audi products. Altera’s FPGAs are opti- mized for sensor fusion, combining data from multiple sen- sors in the vehicle for highly reliable object detection [15]. Similarly, Zynq UltraScale MPSoC is also designed for autonomous driving tasks [16]. When running CNN tasks, it achieves 14 images/s/W, which outperforms the Tesla K40 GPU (4 images/s/W). Also, for object tracking tasks, it reaches 60 frames/s in a live 1080p video stream.

MobilEye EyeQ5 is a leading application-specified inte- grated circuit (ASIC)-based solution for autonomous driving [17]. EyeQ5 features heterogeneous, fully pro- grammable accelerators, where each of the four accelera- tor types in the chip is optimized for their own family of algorithms, including computer-vision, signal-processing, and machine-learning tasks. This diversity of accelerator architectures enables applications to save both computa- tional time and energy by using the most suitable core   for every task. To enable system expansion with multiple EyeQ5 devices, EyeQ5 implements two PCI-E ports for interprocessor communication.

  - Real-Time Operating Systems: A real-time operating system (RTOS) is a special operating system intended to serve real-time applications, such as industrial, automo- tive, aviation, military, and so on [18]. QNX is a popular commercial RTOS widely used in the automotive indus- try. The QNX kernel contains only CPU scheduling, inter- process communication, interrupt redirection, and timers. Everything else runs as a user process, including a special process known as proc that performs process creation

1700 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

and memory management by operating in conjunction with the microkernel [19]. This is achieved by two key mechanisms—subroutine-call-type interprocess communi- cation and a boot loader that loads an image which con- tains not only the kernel but also any desired collection of user programs and shared libraries. There are no device drivers in the kernel.

Another popular commercial RTOS is VxWorks, which is designed for use in embedded systems requiring real- time, deterministic performance and, in many cases, safety and security certification [20]. VxWorks supports mul- tiple architectures including Intel architecture, POWER architecture, and ARM architectures. VxWorks have been used in multicore asymmetric multiprocessing, symmetric multiprocessing, and mixed modes.

Both QNX and VxWorks use real-time kernels for mission-critical applications subject to real-time con- straints, which guaranty a response within predefined time constraints. While QNX is based on a message passing architecture, VxWorks utilizes a shared memory architec- ture. Message passing is fundamental to the kernel design, which allows the system to pass information from one task to another or to several others in the system. Shared memory architecture refers to a system that has its own private address space for physically distributed memories. On kernel design, QNX uses a microkernel, and VxWorks uses a monolithic kernel. A microkernel leverages system calls to manage basic services such as address space man- agement, thread management, and interprocess communi- cations. A monolithic kernel manages all the basic services and user-defined services including interprocess commu- nications in protected kernel space. Therefore, using a

monolithic kernel design, VxWorks is self-contained.

On priority scheduling under QNX, all the processes run on a priority-driven preemptive basis, meaning the process with the highest priority gets to access the CPU first and the priorities range from 0 to 31. The scheduling occurs in real time, and every thread inherits its parent’s priority by default. VxWorks, on the other hand, uses only two types of scheduling algorithms, preemptive priority-based and round-robin scheduling.

  - Middleware: On top of the RTOS, we need a mid- dleware layer to bind different autonomous driving ser- vices together. To achieve this, most existing autonomous driving solutions utilize the robot operating system (ROS)

[21] or modified versions of ROS. Specifically, ROS is a communication middleware that facilitates communica- tions between different parts of an autonomous vehicle system. For instance, the image capture service publishes messages through ROS, and both the localization service and the obstacle detection service subscribe to the pub- lished images to generate position and obstacle updates.

Although ROS is a popular choice for the operating sys- tem in autonomous vehicle systems, in practice, it suffers from a few problems. The first one is reliability; ROS has  a single master and no monitor to recover failed nodes.

Fig. 3. Autonomous driving cloud architecture.

Performance is second; when sending out broadcast mes- sages, it duplicates the message multiple times, leading to performance degradation. Security is also a concern; it has no authentication and encryption mechanisms. Although ROS 2.0 promised to fix these problems, it has not yet been extensively tested, and many features are not yet available. Most importantly, for a piece of software to run securely and reliably in vehicles, it needs to  meet  cer- tain automotive-grade standards, such as ISO26262 [22]. Unfortunately, an automotive-grade middleware is still missing today.

# Cloud Subsystem

Autonomous vehicles are mobile systems that, there- fore, need a cloud subsystem to benefit from the extra computing power of a cloud subsystem. The two main functions provided by the cloud include distributed com- puting and distributed storage. Such a cloud subsystem has several applications, including simulation, which is used to verify new algorithms, HD map production, and deep learning model training [23]. Note that due to the limitations on communication bandwidth, latency, and reli- ability, currently, we only perform offline computing tasks on the cloud. With more advancements in communication infrastructure, we enable more edge-cloud cooperation in real-time computing tasks.

  - Simulation: The first application of a cloud platform system is a simulation. There are two main kinds of simulation technologies. The first type is to simulate the environment based on synthetic data; these simulators are mainly used for the control and planning,  especially at the initial development stage of the algorithm. The second type of simulator is based on real data playback to verify the function and performance of different components, which is mainly used in the iterative process of algorithm development. Take planning and control algorithm devel- opment for example, with offline simulations, we verify the effectiveness of planning and control algorithms, especially with different corner cases. Then, once the algorithms pass all the simulation scenarios, we deploy the algorithms in a vehicle for real-time execution.

As described in [24], the core problem of the  sim- ulator lies in how realistically we simulate the actual driving environment. No matter how good the simulator

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1701

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

is, the artificial simulation of the scene and the real scene still has some differences. There are still many unexpected events in the real scene that cannot be simulated in a sim- ulator. Therefore, if real traffic data are used to reproduce the real scene, better test results will be achieved compared to the artificial simulation of the scene.

However, the major problem of replaying real-world data is the computing power required to process it. In order to reproduce the scene of every section of the real world on the simulator, the autonomous vehicles need to collect the information of each section of the road. This amount of data cannot be processed on single machines. In addition, in each scene, it is further broken down into basic fragments and rearrange the combinations of these fragments to generate more test cases. However, this would generate even more data and add more burden to the simulation platform which is already stressed.

  - HD Map Generation: As detailed in [25], HD  maps are generated in the following steps.

Pose Estimation: For map data collection vehicles, getting accurate poses (location and orientation) of the vehicles is key to generating HD maps. If the poses of the vehicles col- lecting map data are inaccurate, it is impossible to produce precise maps. Once we have the accurate poses of the data- collecting  vehicles,  and  with  accurate  sensor  installation information such as how the sensors are mounted and their relative angles to the vehicle frame, we could calculate the accurate poses of the generated point clouds.

Map Data Fusion: Once we have accurate poses, the next step is map data fusion. Map data here includes LiDAR 3- D  point  clouds  and  camera  images.  In  this  step, multiple scans  of  point  clouds  are  calibrated  and  then  aligned  to generate  denser  point  clouds,  and  the  generated  point clouds  and  captured  images  are  then  registered  to  each other.  This  way,  we  could  use  the  point  clouds  to  get the 3-D location of objects and use the registered images to  extract  semantic  information.  Note  that,  point  clouds accurately provide 3-D positions but usually do not allow semantic information extraction, while on the other hand, images   are   great   for   semantic   information   extraction. By combining point clouds and captured images, we allow both  accurate  poses  extraction  and  semantic  information extraction.

Three-Dimensional  Object  Location  Detection:  For  basic road  elements  whose  geometry  and  precise  locations  are important   (e.g.,   lane   boundaries,   curbs,   traffic   lights, overpasses,  railway  tracks,  guardrails,  light  poles,  speed bumps,  even  potholes,  etc.),  we  need  to  register  their precise 3-D locations and geometries in the HD maps.

Semantics/Attribute    Extraction:    At   last,   we   extract semantics and attributes from captured data and integrate these into the HD Maps. The semantic information includes lane/road model construction, traffic signs recognition and their association with lanes, the association of traffic lights with  lanes,  road  marking  semantics  extraction,  road  ele- ments, and so on.

  - Model Training: The second application this infrastructure needs to support is offline model training. As we use different deep learning models in autonomous driving, it is imperative to provide updates that will continuously improve the effectiveness and efficiency of these models. However, since the amount of raw data generated is enormous, we would not be able to achieve fast model training using single servers.

One   effective   way   to   achieve   high   performance   is through  the  utilization  of  heterogeneous  computing  and high-throughput  storage  systems.  As  described  in  [23], Liu  et   al.   integrated  a  distributed  computing  platform, a  heterogeneous computing  acceleration layer, and  a  dis- tributed  storage  layer  together  to  deliver  an  extremely high-performance  and  high-throughput  distributed  train- ing  engine.  Specifically,  to  synchronize  the  nodes,  at  the end  of  each  training  iteration,  this  system  summarizes all  the  parameter updates  from  each  node,  performs  cal- culations  to  derive  a  new  set  of  parameters,  and  then broadcasts  the  new  set  of  parameters  to  each  node  so they  start  the  next  iteration  of  training.  It  is  the  role  of the  parameter  server  to  efficiently  store  and  update  the parameters.

## I NNO V ATI ONS O N T H E VEH I CU L A R CO MP UTI N G E DGE

In this Section, we review the latest progress in the design of edge computing systems for autonomous driving applications. First, we start with benchmark suites avail- able for evaluating edge computing system designs. Sec- ond, we review different approaches in designing com- puter architectures for autonomous driving workloads. Third, we describe the designs of runtime layers for effi- cient mapping of incoming workloads onto heterogeneous computing units. Fourth, we discuss the designs of middle- ware for binding different autonomous driving functional modules. Last, we present three examples of autonomous driving edge computing systems.

# Benchmark Tools

To improve a computing system, the most effective tool is a standard benchmark suite to represent the workloads widely used in the target applications. The same princi- ple applies when it comes to designing and improving edge computing systems for autonomous vehicles. Cur- rent research in this area is divided into two categories: data sets and workloads. KITTI [26], [27] was the first benchmark data set related to autonomous driving. It is composed of rich vision  sensor  data  with  labels,  such  as monocular/stereo image data and 3-D LiDAR data. According to different data types, it also provides a ded- icated method to generate the ground truth and to cal- culate the evaluation metrics. KITTI was built for evalu- ating the performance of algorithms in the autonomous driving scenario, including but not limited to visual odom- etry, lane detection, object detection, and object tracking.

1702 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

In addition to KITTI, there are some customized bench- mark data sets for each algorithm, such as the Technical University of Munich Red Green Blue-Depth (TUM RGB-

D) [28] for RGB-D simultaneous localization and mapping (SLAM), PASCAL3D [29] for 3-D object  detection,  and the MOTChallenge benchmark [30], [31] for multitarget tracking. These kinds of data sets serve as very good data sources for stressing edge computing systems.

Another class of related benchmark suites is designed to benchmark the performance of novel hardware archi- tectures and software framework, which usually consists of a set of computer  vision  kernels  and  applications.  The San Diego Vision Benchmark Suite (SD-VBS)  [32]  and MEVBench [33] both are performance benchmark suites for mobile computer vision system. SD-VBS provides single-threaded C and MATLAB implementations of nine high-level vision applications. MEVBench is an extended benchmark based on SD-VBS. It provides single-threaded and multithreaded C++ implementations of 15 vision applications. However, these two benchmarks are prior works in the field, so they are not targeted toward hetero- geneous platforms such as GPUs and did not contain novel workloads, such as the deep learning algorithms. SLAM- Bench [34] concentrates on using a complete RGB-D SLAM application to evaluate  novel  heterogeneous  hardware. It takes KinectFusion [35] as the implementation and pro- vided C++, OpenMP, OpenCL, and CUDA versions of key function kernels for heterogeneous hardware. These efforts are a step in the right direction, but we still need a compre- hensive benchmark that contains diverse workloads that cover varied application scenarios of autonomous vehicles (like the MAVBench [126] for micro aerial vehicle system benchmarking) to evaluate the autonomous driving edge computing systems as we mentioned above.

CAVBench is a released benchmark suite specially devel-

oped for evaluating autonomous driving computing system performance [36]. It summarizes four application scenar- ios on CAVs: autonomous driving, real-time diagnostics, in-vehicle infotainment, and third-party applications, and chooses six classic and diverse real-world on vehicle appli- cations as evaluation workloads, which are SLAM, object detection, object tracking, battery diagnostics, speech recognition, and edge video analysis. CAVBench takes four real-world data sets as the standard input to the six work- loads and generates two categories of output metrics. One metric is an application perspective metric, which includes the execution time breakdown for each application, help- ing developers find the performance bottleneck in the application side. Another is a system perspective metric, which is the Quality of Service–Resource Utilization curve (QoS-RU curve). The QoS-RU curve is used to calculate the matching factor (MF) between the application and the computing platform on autonomous vehicles. The QoS- RU curve is considered as a quantitative performance index of the computing platform that helps researchers and developers optimize on-vehicle applications and CAVs computing  architecture. We  hope to see more research  in

the area of benchmarking for autonomous vehicle work- loads, but currently, CAVBench serves as a good starting point to study edge computing systems for autonomous driving.

As autonomous driving  is  still  a  fast  developing field, we hope to see continuous effort to  incorporate more dynamic workloads and data to cover emerging autonomous driving usage scenarios. In addition, stan- dardized scoring methods are required (but still missing) to rank different edge computing systems based on differ- ent optimization metrics.

# Computing Architectures

Once  we  have  standard  benchmark  suites,  we  start developing suitable  architectures  for autonomous  driving workloads.  Liu  et  al.  [1]  proposed  a  computer  architec- ture  for  autonomous  vehicles  which  fully  utilizes  hybrid heterogeneous hardware. In this work, the applications for autonomous  driving  are  divided  into  three  stages:  sens- ing,  perception,  and  decision-making.  The  authors  com- pared the performance of different hardware running basic autonomous driving tasks and concluded that localization and  perception as  the bottlenecks of  autonomous  driving computing  systems, and  they also identified the  need for different  hardware  accelerators  for  different  workloads. Furthermore,   the  authors   proposed   and   developed  an autonomous driving computing architecture and software stack that is modular, secure, dynamic, high-performance, and energy-efficient. By fully utilizing heterogeneous com- puting  components,  such  as  CPU,  GPU,  and  DSP,  their prototype system on an ARM Mobile SoC consumes 11 W on average and is able to drive a mobile vehicle at 5 mph. In addition, the authors indicated that with more comput- ing  resources,  the  system would  be  able  to  process  more data and would eventually satisfy the need of a production- level autonomous driving system.

Similarly,   Lin   et   al.   [37]   explored   the   architectural constraints  and  acceleration  of  the  autonomous  driving system. The authors presented and formalized the design constraints of autonomous driving systems in performance, predictability, storage, thermal, and power when building autonomous  driving  systems.  To  investigate  the  design of  the  autonomous  driving  systems,  the  authors  devel- oped an end-to-end autonomous driving system based on machine  learning  algorithmic  components.  Through  the experiments  on  this  system,  the  authors  identified  three computational   bottlenecks,   namely,   localization,   object detection,  and  object  tracking.  To  design  a  system which meets all the design constraints, the authors also explored three  different  accelerator  platforms  to  accelerate  these computational bottlenecks. The authors demonstrated that GPU,  FPGA,  and  ASIC-accelerated  systems  could  effec- tively  reduce  the  tail  latency  of  these  algorithms.  Based on these acceleration systems, the authors further explored the  tradeoffs  among  performance,  power,  and  scalability of  the  autonomous  driving  systems.  Their  conclusion  is

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1703

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

that although power-hungry accelerators like GPUs pre- dictably deliver the computation at low latency,  their  high power consumption, further magnified by the cool- ing load to meet the thermal constraints, significantly degrades the driving range and fuel efficiency of the vehicle. Finally, the authors indicated that computational capability remains the bottleneck that prevents them from benefiting from the higher system accuracy enabled by higher resolution cameras.

Interestingly,  in  their  pioneering  architectural  explo- ration work, the authors discussed above both concluded that localization and perception are the computing bottle- necks and heterogeneous computing is a feasible approach to  accelerate  these  workloads.  For  localization  accelera- tion, Tang et  al.  [38] proposed a heterogeneous architec- ture for SLAM [38]. The authors first conducted a thorough study  to  understand  visual  inertial  SLAM  performance and energy consumption on existing heterogeneous SoCs. The  initial   findings   indicate  that  existing  SoC  designs are  not  optimized  for  SLAM  applications,  and  systematic optimizations are required in the IO interface, the memory subsystem, as well as computation acceleration. Based on these findings, the authors proposed a heterogeneous SoC architecture  optimized  for  visual  inertial  SLAM  applica- tions. Instead of simply adding an accelerator, the authors systematically  integrated  direct  IO,  feature  buffer,  and  a feature  extraction  accelerator.  To  prove  the  effectiveness of  this  design,  the  authors  implemented  the  proposed architecture on a Xilinx Zynq UltraScale MPSoC and were able to deliver over 60 frames/s performance with average power  less  than  5  W.  These  results  verify  that  the  pro- posed architecture is capable of achieving performance and energy consumption optimization for visual inertial SLAM applications.

Similarly, to solve the localization computing problem, Zhang et al. [39] proposed an algorithm and hardware codesign methodology for visual–inertial odometry (VIO) systems, in which the robot estimates its ego-motion (and a landmark-based map) from the onboard camera and IMU data. The authors argued that scaling down VIO to miniaturized platforms (without sacrificing performance) requires a paradigm shift in the design of perception algo- rithms, and the authors advocated a codesign  approach  in which algorithmic and hardware design choices are tightly coupled. In detail, the authors characterized the design space by discussing how a relevant set of design choices affects the resource-performance tradeoff in VIO. Also, the authors demonstrated the result of the codesign process by providing a VIO implementation on specialized hardware and showing that such implementation has the same accuracy and speed of a desktop implementation while requiring a fraction of the power.

Besides  academic   research,   PerceptIn   has   released a commercial production SLAM system titled  DragonFly+ [40]. DragonFly+ is an FPGA-based real-time localization   module   with   several   advanced   features:

1)   hardware   synchronizations   among   the   four image

channels as well as the IMU; 2) a  direct IO architecture   to reduce off-chip memory communication; and 3) a fully pipelined architecture to accelerate the image processing frontend. In addition, parallel and multiplexing processing techniques are employed to achieve a good balance between bandwidth and hardware resource consumption. Based on publicly available data, for processing  four-  way 720p images, DragonFly+ achieves 42 frames/s performance  while  consuming  only  2.3  W  of  power.  In comparisons, Nvidia  Jetson  TX1  GPU  SoC  achieves  9 frames/s at 7 W and Intel Core i7 achieves 15 frames/s at 80 W. Therefore, DragonFly+ is 3× more power efficient and delivers 5× of computing power compared to Nvidia TX1, and 34× more power efficient and delivers 3× of computing power compared Intel Core i7.

For  perception  acceleration,  most  recent  research  has focused on  the  acceleration of  deep convolutional  neural networks  (CNNs).  To  enable  CNN  accelerators  to  sup- port  a  wide  variety  of  different  applications  with  suffi- cient  flexibility  and  efficiency,  Liu  et  al.  [41]  proposed  a novel domain-specific instruction set architecture (ISA) for neural  network  accelerators. The proposed  ISA  is  a  load- store  architecture  that  integrates  scalar,  vector,  matrix, logical, data transfer, and control instructions, based on a comprehensive  analysis  of  existing  neural  network  accel- eration  techniques.  The  authors  demonstrated  that  the proposed  ISA  exhibits  strong  descriptive  capacity  over  a broad range of neural network acceleration techniques and provides  higher  code  density  than  general-purpose  ISAs such as x86,  Microprocessor without  Interlocked Pipeline Stages (MIPS), and GPGPU.

Realizing that data movement is a key bottleneck for CNN computations, Chen et al. [42] presented a dataflow to minimize the energy consumption of data movement on a  spatial architecture. The  key is  to  reuse local data  of filter weights and feature map pixels,  or  activations,  in the high-dimensional convolutions, and minimize data movement of partial sum accumulations. The proposed dataflow adapts to different CNN shape configurations and reduces all types of data movement by maximally utilizing processing engine (PE) local storage, spatial parallelism, and direct inter-PE communication. Through the CNN configurations of AlexNet, evaluation experiments show that the proposed dataflow for  more  energy  efficient than other dataflows for both convolutional and fully connected layers.

Also, memory  access latency and throughput  are  often bottlenecks  for  neural  network  computations.  One  tech- nique  that  has  the  potential  to  be  used  for  the  main memory is the metaloxide resistive random access memory (ReRAM).  Moreover,  with  the  crossbar  array  structure, ReRAM  performs  matrix–vector  multiplication  more  effi- ciently  and  has  been  widely  studied  in  accelerations  of CNN  applications.  To  accelerate CNN  computations  from the  memory  side,  Chi et al. [43]  proposed  a  processor- in-memory  (PIM)  architecture  to  accelerate  CNN  appli- cations  in  ReRAM  based  main  memory.  In  the  proposed

1704 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

design, a portion of ReRAM crossbar arrays is configured as accelerators for CNN applications or as normal memory for larger memory space. The microarchitecture and cir- cuit designs are provided enable the morphable functions with an insignificant area overhead. Benefiting from both the PIM architecture and the efficiency of using ReRAM for CNN computation, the proposed design achieved sig- nificant performance improvement and energy savings, demonstrating the effectiveness of CNN accelerations from the memory side.

We have seen much recent progress on acceleration localization and perception functions, with many  stud- ies focused on reconfigurable  fabrics  [44]–[48];  how- ever, with limited chip area, it is inefficient to integrate one accelerator for each perception and localization task. Therefore, the efficient utilization of reconfigurable fab- rics is a major challenge for autonomous driving edge computing  design. For  instance, the object tracking task  is triggered by the object recognition task and the traffic prediction task is triggered by the object tracking task. The data uploading task is also not needed all the time since uploading data in batches usually improves throughput and reduces bandwidth usage. To optimize for chip area and power consumption, one way is to have these tasks time-share an FPGA chip in the system. It has been demon- strated that using partial-reconfiguration techniques [49], an FPGA soft core could be changed within less than a few milliseconds, making time-sharing possible in real time.

In the near future, as more autonomous driving work- loads and usage scenarios emerge, we look forward to the designs of more accelerators targeted for these workloads. Also, we expect to see more exploration studies on the cache, memory, and storage architectures for autonomous driving workloads. In addition, hardware security for autonomous driving is of utmost importance. Within a decade, the research community and the industry shall be able to come up with a “general-purposed” architecture design for autonomous driving workloads.

# Runtime Systems

With heterogeneous architectures ready for autonomous driving tasks, the next challenge is how to dispatch incoming tasks to different computing units at runtime to achieve optimal energy efficiency and performance. This is achieved through a runtime layer. Designing runtime for heterogeneous autonomous driving systems is a whole new research area with tremendous potentials, as most existing runtime designs focus on either mapping one algorithm to one type of accelerators or on scheduling for homogeneous systems or heterogeneous systems with a single accelerator.

Several  existing  designs  focus  on  mapping  one  deep learning  or  computer  vision  workload  to  heterogeneous architectures:  Hegde  et   al.   [50]  propose  a  framework for  easy  mapping  of  CNN  specifications  to  accelerators such as FPGAs, DSPs, GPUs, and Reduced Instruction Set Computer  (RISC)   multicores.  Malik et al. [51]   compare

the   performance   and   energy   efficiency   of   computer vision  algorithms  on  on-chip  FPGA  accelerators and  GPU accelerators. Many studies have explored the optimization of  deep  learning  algorithms  on  embedded  GPU  or  FPGA accelerator   [52],   [53].   There   have   also   been   many projects   on   optimizing   computer   vision   related   tasks on  embedded  platforms.  Honegger  et   al.   [54]  propose FPGA  acceleration  of  embedded  computer  vision.  Satria et   al.   [55]   perform   platform-specific   optimizations   of face   detection   on   embedded   GPU-based  platform   and reported   real-time   performance.   Vasilyev   et    al.    [56] evaluate  computer  vision  algorithms  on  programmable architectures. Nardi et  al.  [57] present a benchmark suite to  evaluate  dense  SLAM  algorithms  across  desktop  and embedded  platforms  in  terms  of  accuracy,  performance, and energy consumption.  However, these designs did not consider the complexity of integrating the various kinds of workloads  into  a  system and  only  focus  on  mapping  one task to different accelerators.

Other  existing  designs  focus  on  scheduling  for  hetero- geneous  architectures with one  accelerator that has been broadly  studied  for  single-ISA  multiprocessors,  such  as asymmetric  multicore  architectures,  i.e.,  big  and  small cores,  and  multi-ISA  multiprocessors  such  as  CPU  with GPU.  On  the  single-ISA  multiprocessor  side,  much  work has been done at the operating system level to map work- load onto most appropriate core type in run time. Koufaty et  al.  [58]  identified  that  the  periods  of  core  stalls  is  a good  indicator  to  predict  the  core  type  best  suited  for an  application.  Based on  the  indicator,  a  biased schedule strategy was added to operating systems to improve system throughput. Saez et al. [59] proposed a scheduler that adds efficiency specialization and thread-level parallelism (TLP) specialization to operating systems to optimize throughput and power at the same time. Efficient specialization maps CPU-intensive  workloads  onto  fast  cores  and  memory- intensive  workloads  onto  slow  cores.  TLP  specialization uses fast cores to accelerate a sequential phase of parallel applications  and  use  slow  cores  for  the  parallel phase  to achieve  energy  efficiency.  On  the  asymmetric  multicore architectures side, Jiménez et al. [60] proposed a user-level scheduler for CPU with GPU like system. It evaluates and records the performance of a process on each PE at the ini- tial phase. Then, based on this history information, it maps the  application  onto  the  best  suited  PE.  Luk et al. [61] focus on improving the latency and energy consumption of a single process. It uses dynamic compilation to character- ize workloads, determines optimal mapping, and generates codes for CPUs and GPUs.

Unlike existing runtime designs, Liu et al. [62] proposed PerceptIn  Runtime  (PIRT),  the  first  runtime  framework that  is  able  to  dynamically  map  various  computer  vision and deep learning workloads to multiple accelerators and to the cloud. The authors first conducted a comprehensive study  of  emerging  robotic  applications  on  heterogeneous SoC   architectures.   Based   on   the   results,   the   authors designed  and  implemented  PIRT  to  utilize  not  only  the

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1705

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

on-chip heterogeneous computing resources but also the cloud to achieve high performance and energy efficiency. To verify its effectiveness, the authors have  deployed PIRT on a production mobile robot to demonstrate that full robotic workloads, including autonomous navigation, obstacle detection, route planning, large map generation, and scene understanding, are efficiently executed simultaneously with 11 W of power consumption.

The runtime layer connects autonomous driving soft- ware and hardware, but there are several upcoming chal- lenges in the design of runtime systems for autonomous driving. First, as the computing system becomes more heterogeneous, the runtime design becomes more com- plicated in order to dynamically dispatch incoming work- loads. Second, as more edge clouds  become  available,  the runtime system needs to be cloud-aware and able to dispatch workloads to edge clouds. Third, the runtime shall provide a good abstraction to hide all the low-level implementations.

# Middleware

Robotic systems, such as autonomous vehicle systems, often involve multiple services, with a lot of dependencies in between. To facilitate the complex interactions between these services, to simplify software design, and to hide  the complexity of low-level communication and the het- erogeneity of the sensors, a middleware is required.

An early design of robotic middleware is Miro, a distrib- uted object-oriented framework for mobile robot control, based on Common Object Request Broker Architecture technology (COBRA) [63]. The mirocore components have been developed under the aid of Adaptive Communications Environment, an object-oriented multiplatform framework for OS-independent interprocess, network, and real-time communication. Mirocore provides generic abstract ser- vices like localization or behavior engines, which is applied on different robot platforms. Miro supports several robotic platforms including Pioneers, B21, robot soccer robots, and various robotic sensors.

ORCA is an open-source component-based software engineering framework developed for mobile robotics with an associated repository  of  free,  reusable  components for building mobile robotic systems [64]. ORCA’s project goals include enabling software reuse by defining a set of commonly-used interfaces; simplifying software reuse by providing libraries with a high-level convenient application program interface (API); and encouraging software reuse by maintaining a repository of components.

Urbi is open-source cross-platform software used to develop applications for robotics and complex systems [65]. Urbi is based on the UObject distributed C++ component architecture. Urbi includes the urbiscript orchestration language, a parallel and event-driven script language. In this design, UObject components are plugged into urbiscript as native objects to specify their interactions and data exchanges. UObjects are linked to the urbiscript

interpreter, or executed as autonomous processes in “remote” mode, either in another thread, another process, a machine on the local network, or a machine on  a  distant network.

RT-middleware is a common platform standard for dis- tributed object technology based robots based on dis- tributed object technology [66]. RT-middleware supports the construction of various networked robotic systems through the integration of various network-enabled robotic elements called RT-components. In the RT-middleware, robotics elements, such as actuators, are regarded as RT- components, and the whole robotic system is constructed by connecting these RT-components. This distributed archi- tecture helps developers to reuse the robotic elements and boosts the reliability of the robotic system.

OpenRDK is an open-source software framework for robotics for  developing  loosely  coupled  modules  [67].  It provides transparent concurrency management, inter- process via sockets, and intraprocess via shared memory. Modules for connecting to simulators and generic robot drivers are provided.

The above-mentioned middleware projects mostly focused on providing a software component management framework for mobile robots and they were not used in autonomous vehicles. On the other hand, ROS has been widely used in autonomous vehicle development [21], mainly due to the popularity of ROS amount robotic devel- opers and the richness of its software packages. However, as discussed in Section II, at its current state, ROS is not suitable for the production deployment of autonomous vehicles as it suffers from performance, reliability, and security issues.

PerceptIn Operating System (PIOS) is a developed extremely light-weighted middleware to facilitate the com- munications between services on production low-speed autonomous vehicles [68]. PIOS builds on top of Nanomsg, a networking library written in C that allows for easy integration of shared memory, transmission control pro- tocol (TCP)/IP, in-process messaging, and web sockets while  retaining efficiency [69].  Compared to ROS, PIOS  is extremely lightweight, able to achieve a startup memory footprint of merely 10 KB, which is negligible comparing to 50 MB of ROS startup footprint.

OpenEI [70] is a lightweight software platform to equip edges with intelligent processing and data sharing capa- bility. Due to the computing power limitation, weak data sharing, and collaborating, the deployment of artificial intelligence (AI)-based algorithms has faced many chal- lenges. The goal of OpenEI is  that any  hardware,  such  as Raspberry Pi, will become an intelligent edge after deploying OpenEI.

The middleware layer facilitates communication between different autonomous driving services, but we summarize several challenges. First, the middleware should impose minimal computing overhead and memory footprint, thus making it scalable. Second, as some autonomous driving services may stay in edge clouds,

1706 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

Table 1 Summary of Computing Edge Designs

the middleware should enable  a  smooth  edge  client  and cloud communication. Third and most importantly, the middleware should be secure and reliable to guarantee the quality of service and autonomous vehicle safety.

# Case Studies

To simultaneously enable multiple autonomous  driving services,  including  localization,  perception,  and  speech recognition  workloads  on  affordable  embedded  systems, Tang et al. designed and implemented Π-Edge, a complete edge  computing  framework  for  autonomous  robots  and vehicles  [68].  The  challenge  of  designing  such  a  system include  the  following:  managing  different  autonomous driving  services  and  their  communications  with  minimal overheads,  fully  utilizing  the  heterogeneous  computing resources on the edge device, and offloading some of the tasks to the cloud for energy efficiency.

To achieve these, first, the authors developed a run- time layer to fully utilize the heterogeneous computing resources of low-power edge computing systems; second, the authors developed an extremely lightweight middle- ware to manage multiple autonomous driving services and their communications; third, the authors developed an edge-cloud coordinator to dynamically offload tasks to the cloud to optimize client system energy consumption.

OpenVDAP is another real-world edge computing system which is a full-stack edge-based platform including vehicle computing unit, an isolation-supported and security and privacy-preserved vehicle operation system, an edge-aware application library, as well as task offloading and scheduling strategy [71]. OpenVDAP allows connected and autonomous vehicles (CAVs) to dynamically examine each task’s status, computation cost, and the optimal scheduling method so that each service could be finished in near real time with low overhead. OpenVDAP is featured as a two-tier architecture via a series of systematic mechanisms that enable CAVs to dynamically detect service status and to identify the optimal offloading des- tination so that each service could be finished at the right time. In addition, OpenVDAP offers an open and free edge- aware library that contains how to access and deploy edge- computing-based vehicle applications and various common used AI models, thus enabling researchers and developers to deploy, test, and validate their applications in the real

environment. Novel applications such as AutoVAPS benefit from it [72]. AutoVAPS is an Internet-of-Things (IoT)- enabled public safety service, which integrates body-worn cameras and other sensors on the vehicle for public safety. HydraOne is an indoor experimental research and education platform for edge computing in CAVs scenar- ios [127]. HydraOne is a full-stack research and education platform from hardware to software, including mechanical components, vision sensors, as well as computing and com- munication system. All resources on HydraOne are man- aged by the Robot Operating System [76], and the data analytics on HydraOne is managed by OpenVDAP, which is a full-stack edge based platform [71]. HydraOne has three key characteristics: design modularization, resource extensibility, and openness, as well as function isolation, which allows users to conduct various research and educa-

tion experiments of CAVs on HydraOne.

## I NNO V ATI ONS O N T H E V2 X IN F R A S T R U C TU RE

One effective method to alleviate the huge computing demand on autonomous driving edge computing systems is V2X technologies. V2X is defined as a vehicle commu- nication system that consists of many types of communica- tions: V2V, vehicle-to-network (V2N), vehicle-to-pedestrian (V2P), V2I, vehicle-to-device (V2D), and vehicle-to-grid (V2G). Currently, most research focuses on V2V and V2I. While conventional autonomous driving systems require costly sensors and edge computing equipment within the vehicle, V2X takes a different and potentially complimen- tary approach by investing in road infrastructure, thus alleviating the computing and sensing costs in vehicles.

With the rapid deployment of edge computing facilities in the infrastructure, more and more autonomous driving applications have started leveraging V2X communications to make the in-vehicle edge computing system more effi- cient. One popular direction is cooperative autonomous driving. The cooperation of autonomous driving edge computing system with V2X technology makes it pos- sible to build a safe and efficient autonomous driving system [73]. However, the  application  and  deployment of cooperative autonomous driving systems are still open research problems. In this section, we discuss the evolution of V2X technology and present four case studies of V2X

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1707

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

for autonomous driving: convoy driving, cooperative lane change, cooperative intersection management, and coop- erative sensing.

# Evolution of V2X Technology

In the development of V2X technology, many researchers have contributed solutions to specific challenges of V2X communication protocols. The intervehicle hazard warn- ing (IVHW) system is one of the earliest studies to take the idea of improving vehicle safety based on communication [74]. The project is funded by the German Ministry of Edu- cation and Research and the French government. IVHW  is a communication system in which warning messages are transmitted as broadcast messages in the frequency band of 869 MHz [75]. IVHW takes a local decision- making strategy. After the  vehicle  receives the  message, it will do relevant checks to decide whether the warning message is relevant and should be shown to the driver. The majority of the research efforts have been made on the design of relevance check algorithms. However, as IVHW takes a broadcast mechanism to  share  the  message,  there is a huge waste in both bandwidth and computing resources.

Compared with the broadcast message in IVHW, ad hoc networking is a better solution to support multihop inter- vehicle communication [76]. FleetNet is another research project taking the idea of vehicle communication [77], and it is based on ad hoc networking. In addition, the FleetNet project  also  wants  to  provide  a  communication  platform for Internet-protocol-based applications. FleetNet is imple- mented  based  on  the  IEEE  802.11  Wireless  LAN  system [78].  For  V2V  communication,  if  two  vehicles  are  not directly connected wirelessly, it would need other vehicles to  forward  the  message  for  them.  Designing  the  routing and  forwarding  protocol  is  a  major  challenge.  In  order to  meet  the  requirements  for  adaptability  and  scalability, FleetNet  proposed  a  position-based  forwarding  mecha- nism.  The  idea  is  to  choose  the  next  hop  to  forward  the message based on the geographical location of the vehicle. CarTALK 2000 is also a project working on using an ad hoc communication network to support co-operative driver assistance  applications  [79].  There  is  a  major  challenge for  ad  hoc-based  routing  in  V2V  communication  because the vehicle network topology is dynamic and the number of  vehicles is  frequently changing  [80].  In  order  to  solve the problem, a spatial aware routing algorithm is proposed in  CarTALK  2000,  which  takes  spatial  information  like underlying  road  topology  into  consideration.  Compared with FleetNet, CarTALK 2000 achieves better performance as it uses spatial information as additional input for routing algorithm.  Another  similar  part  of  CarTALK  2000  and FleetNet is that they are both based on WLAN technology. AKTIV is another project that is the first one tried to apply cellular  systems  in  driving  safety  applications  [81].  One of  the  reasons  that  FleetNet  and  CarTALK  2000  project built  their  system based on  WLAN  technology  is  that  the

latency of safety-related applications required is less than 500 ms. However, with the assumption that a long-term evolution (LTE) communication system is greatly further developed, cellular systems are a good choice for sparse vehicle networking.

Meanwhile, several research projects have focused on warning applications based on V2V communication. Wire- less local danger warning (WILLWARN) proposed a risk detection approach based on in-vehicle data. The warning message includes obstacles, road conditions, low visibility, and construction sites [82]. Unlike other projects focus-  ing on the V2X technology itself, WILLWARN focuses on enabling V2X technology in some specific scenario such  as the danger spot. Suppose some potential danger is detected in a specific location, but there is no vehicle within the communication range that supports the V2X communication technology to share the warning mes- sage [83]. To share warning messages, WILLWARN pro- posed a decentralized distribution algorithm to transmit the warning message to vehicles approaching the danger spot through V2V communication. The project Network on Wheels (NoW) was one  work  which  takes  the  idea of FleetNet to build vehicle communication based on

802.11  WLAN  and  ad  hoc  networking  [84].  The  goal  of NoW is to set up a communication platform to support both mobility  and Internet applications. For example, a hybrid forwarding  scheme  considering  both  network  layer  and application layer is developed. Also, security and scalability issues are discussed in NoW.

As the infrastructure also plays a very important part  in V2X technology, several efforts focus on building safety applications based on the cooperation with infrastructure. SAFESPOT is an integrated project that is aimed at using roadside infrastructure to improve driving safety [85]. Through combining information from the on-vehicle sen- sors and infrastructure sensors, SAFESPOT detects danger- ous situations and shares the warning messages in real time. Also, the warning forecast is improved from the milliseconds level to the seconds level, thus giving the driver more time to prepare and take action. Five appli- cations are discussed in SAFESPOT, including hazard and incident warning, speed alert, road departure prevention, cooperative intersection collision prevention, and safety margin for assistance and emergency vehicles [86].

In 2007, a nonprofit organization called the Car 2 Car Communication Consortium (C2C-CC) was set up to com- bine all solutions from different former  projects to  make a standard for V2X technology. Since 2010, the focus of work on V2X technology has moved from research topics to the real environment deployment of the whole intelligent transportation system (ITS). One of the most popular deploy projects is simTD [73], targeted on testing the V2X applications in a real metropolitan field. In simTD, all vehicles connect with each other through dedicated short- range communications (DSRC) technology that is based on IEEE 802.11p. Meanwhile, vehicles also communicate with roadside infrastructure using IEEE 802.11p. The system

1708 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

Table 2 Evolution of V2X Communication Technology

architecture of simTD is divided into three parts: the ITS vehicle station, the ITS roadside station, and the ITS central station. Applications for testing in simTD include traffic situation monitoring, traffic flow information and navigation, traffic management, driving assistance, local danger alert, and Internet-based applications.

Cellular V2X (C-V2X) is designed as a unified connec- tivity platform which provides low-latency V2V and V2I communications [87]. It consists of two modes of commu- nications. The first mode uses direct communication links between vehicles, infrastructure, and pedestrian. The sec- ond mode relies on network communication, which lever- ages cellular networks to enable vehicles to receive infor- mation from the Internet. C-V2X further extends the com- munication range of the vehicle and it supports the higher capacity of data for information transmission for vehicles.

# Cooperative Autonomous Driving

Cooperative autonomous driving is divided into two categories: one is cooperative sensing and the other is cooperative decision [88]. Cooperative sensing focuses on sharing sensing information between V2V and V2I. This data sharing increases the sensing range of autonomous vehicles, making the system more robust. The cooperative decision enables a group of autonomous vehicles to coop- erate and make decisions.

Several studies have focused on the exploration of applications for cooperative autonomous driving. In [88], four use cases including convoy driving, cooperative lane change, cooperative intersection management, and coop- erative sensing are demonstrated. According to the design of AutoNet2030 [89], a convoy is formed of vehicles on multilanes into a group and the control of the whole group is decentralized. The safety and efficient control of the convoy require high-frequency exchanges of each vehicle’s dynamic data. As shown in Fig. 4, a roadside edge server

and a cloud server are used to coordinate and manage the vehicles and convoys to go through crossroads safely. One convoy control algorithm in [90] only exchanges dynamic information of the nearby vehicle rather than for all the vehicles within a convoy. This design makes the algorithm easy to converge.

Cooperative lane change is designed to make  vehi-  cles or convoys to collaborate when changing lanes. Proper cooperative lane change cannot only avoid traffic accidents but it  also reduces traffic congestion [91].  MOBIL [92]  is a general model whose objective is to minimize overall braking induced by lane changes.

Cooperative intersection is also helpful for safe driving and traffic control. The World’s Smartest Intersection in Detroit [93] focuses on safety and generates data that

Fig. 4. V2X communications in crossroads.

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1709

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

pinpoint areas where traffic-related fatalities and injuries are reduced. Effective cooperative intersection manage- ment is based on the coordination mechanism between V2V and V2I.

Cooperative sensing increases the autonomous vehicle sensing range through V2X communication. Meanwhile, cooperative sensing also helps in cutting the cost of build- ing autonomous driving. As vehicles rely more on the sensors deployed on roadside infrastructure, the cost of on-vehicle sensors is reduced. In the future, sensor infor- mation may become a service to the vehicle provided by the roadside infrastructure.

V2X networking infrastructure is also a very impor- tant aspect of cooperative autonomous driving. Hetero- geneous vehicular network (HetVNET) [94] is an initial work on networking infrastructure to meet the communi- cation requirements of the ITS. HetVNET integrates LTE with DSRC [95] because relying on the single wireless access network cannot provide satisfactory services in dynamic circumstances. In [96], an improved protocol stack is proposed to support multiple application scenar- ios of autonomous driving in HetVNET. In the protocol, the authors redefined the control messages in HetVNET to support autonomous driving.

Similarly, the vehicular delay-tolerant network (VDTN)

[97]  is  an  innovative  communication  architecture, which is  designed  for  scenarios  with  long  delays  and  sporadic connections.  The  idea  is  to  allow  messages  to  be  for- warded  in  short-range  WiFi  connections  and  reach  the destination  asynchronously.  This  property  enables  VDTN to  support  services  and  applications  even  when  there  is no  end to end  path in  current  Vehicular Ad hoc  Network (VANET).  Dias   et   al.   [98]   discuss   several  cooperation strategies  for  VDTN.  The  challenge  for  cooperation  in VDTN is how to coordinate the vehicle node to share their constrained bandwidth, energy resources, and storage with one  another.  Furthermore,  an  incentive  mechanism  that rewards  or  punishes  vehicles  for  cooperative  behavior  is proposed.

In order to support seamless V2X communication, han- dover  is  also  a  very  important  topic  for  V2X  networking infrastructure.  Due  to  the  dynamic  changing  of  the  net- working  topology  and  the  relatively  small  range  of  the communication  coverage,  the  handover  mechanism  in  a cellular network is no longer suitable for VANET. Based on proactive resource allocation techniques, Ghosh et al. [99] propose a new handover model for VANET. With the help of  proactive  handover,  cooperative  services  are  migrated through  RSUs  with  the  moving  of  the  vehicle.  Hence, proper designing of proactive handover and resource allo- cation  is  essential  for  developing  reliable  and  efficient cooperative systems.

The development of edge computing in the automotive industry is also very inspiring. Automotive Edge Comput- ing Consortium (AECC) is a group formed by automotive companies to promote edge computing technologies in future automobiles [100]. According to an AECC white

paper from 2018, the service scenarios include  intelli-  gent driving, HD map, V2Cloud cruise assist, and several extended services like finance and insurance. In addition, the white paper discusses the service requirements in terms of data source, the volume of data generated in the vehicle, target data traffic rate, response time, and required availability.

# Challenges

In order to guarantee the robustness and safety of autonomous driving systems, autonomous vehicles are typically equipped with numerous expensive sensors and computing systems, leading to extremely high costs and preventing ubiquitous deployment of autonomous vehi- cles. Hence, V2X is a viable solution in decreasing the costs of autonomous driving vehicles as V2X enables information sharing between vehicles and computation offloading to RSUs. There are several challenges in achieving coopera- tive autonomous driving. Here, we discuss the challenges and our vision for application scenario of cooperative decision and cooperative sensing.

  - Cooperative Decision: The challenge of cooperative decisions is handling the dynamic changing topology  with a short-range coverage of V2X communications. The design of VDTN is a good hint to solve this challenge. Effective proactive handover and resource allocation are potential solutions. Also, the coming 5G wireless com- munication [101] also provides a way to handle this challenge.

  - Cooperative Sensing: The main challenge of coopera- tive sensing is sharing the information from infrastructure sensors to autonomous vehicles in real time, and the other challenge is to dynamically trade off the cost of infrastruc- ture sensors and on-vehicle sensors. For the first chal- lenge, the promising edge computing technology is used to solve the problem [71] because edge computing enables the edge node(vehicle) and edge server(infrastructure) to conduct computation and compression to provide real- time performance. In addition, the tradeoff of cost on infrastructure sensors and on-vehicle sensors will be deter- mined by the automobile market. Both the government and companies will invest much money to support edge intelligence.

## VE HICU LAR E DGE S E C U R I T Y

The previous sections reviewed innovations in an edge computing infrastructure to make autonomous driving computing  more  efficient  in  terms   of   performance and energy consumption. As mentioned above, each autonomous vehicle is equipped with or supported by dozens of computing units in the edge and cloud to process the sensor data, to monitor the vehicles’ status and to control the mechanical components. Hence, the security threats against these computing units are of paramount concern. Specifically, the attacks targeting autonomous

1710 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

vehicles could cause terrible traffic accidents, threatening both personal and public safety. In this Section, we review recent advancements in the security of autonomous vehi- cles, including sensor security, operating system security, control system security, and communication security. These security problems cover different layers of the autonomous driving edge computing stack.

# Sensors Security

The autonomous vehicles are equipped with various sen- sors (camera, GNSS, LiDAR, etc.) to enable the perception of the surrounding environments. The most direct security threats against autonomous vehicles are attacks against the sensors. With this attack method, attackers generate incorrect messages or completely block sensor data so as to interfere in the autonomous driving without hacking into the computing system. According to the working principle of sensors, the attackers have many specific attack methods to interfere, blind, or spoof each of them [102].

A camera is the basic visual sensor in autonomous driving systems. Modern autonomous vehicles are usually equipped with multiple cameras with the same or different lenses [1], [26]. In general, many autonomous driving perception workloads take camera images as inputs, for example, object detection and object tracking. The attack- ers place fake traffic lights, traffic signs, and traffic objects (cars or pedestrians) to spoof autonomous  vehicles and let them make the wrong decisions [103].  The  cameras are also interfered with infrared so the attackers use high- brightness IR laser to blind the cameras, thus preventing these cameras from providing effective images for the perception stage [103], [104].

Autonomous vehicles use GNSS and inertial navigation system (INS) sensors to update vehicles’ real-time loca- tions. Typical attacks against GNSS sensors are jamming and spoofing. The attackers could use out-of-band or in- band signals to intentionally interfere with the function of the GNSS receiver [105]. They also deploy GNSS trans- mitter near the autonomous vehicles to deceive the GNSS receiver by replicating original signals and providing false locations [103], [105]. In addition, the INS sensors are sensitive to magnetic fields, so an extra and powerful magnetic field could effectively interfere with the INS sensors to produce incorrect orientation of the vehicles under attack.

LiDAR provides point clouds of the vehicle’s surround- ings to generate 3-D sensing data of the environments. LiDAR measures distance to  a  target  by  illuminating  the target with pulsed laser light and measuring the reflected pulses. A smart surface that is absorbent or reflec- tive deceives LiDAR sensors to miss real obstacles in traffic [102], and light laser pulse illuminating the LiDAR could also manipulate the data sensed by the LiDAR, deceiving the LiDAR to sense objects in incorrect posi- tions and distances [103]. For ultrasonic sensors and radars, which are mostly used for passive perception and the last line of defense for the autonomous vehicles,

Yan et al. [106]  have  successfully  spoofed  and  jammed these  two  kinds  of  sensors  in  the  Tesla  Autopilot  system via the specific signal generator and transmitter.

# Operating Systems Security

One widely used autonomous vehicle operating system is ROS. Attackers target ROS nodes and/or ROS messages. In the ROS running environment, there is no authentica- tion procedure for message passing and new node creation. The attackers use the IP addresses and ports on the master node to create a new ROS node  or  hijack  an  existing  one without further authentication [107].  If  service  on the node keeps consuming system resources, for example, memory footprint or CPU utilization, it will impact the performance of other normal ROS nodes, even crashing the whole autonomous driving system. The attackers also use the controlled ROS node to send manipulated messages to disturb other nodes running and output.

For the attacks on ROS messages, the first security threat is message capture. The attackers monitor and record every ROS message topics via the IP address and port on the mas- ter node. The recorded data are stored in the ROS bag file, and the attackers play the ROS bag file to resend history ROS messages, which will affect current ROS messages communication [107]. The message passing mechanism of ROS is based on the socket communication so the attackers sniff the network packets to monitor and intercept the ROS messages remotely without hacking in the master node [108], [109]. The attacks on  ROS  messages do  not  need to start or hijack a ROS node, but the security threat level is not lower than the attack with the ROS node method.

# Control Systems Security

In modern vehicles, many digital devices and mechanical components are controlled by electronic control units (ECUs). The ECUs are connected to each other via the digital buses, which form the in-vehicle network. Controller area network (CAN) is the primary bus protocol in the vehicle [110]. CAN is the typical bus topology; there is no master/slave node concept in the CAN bus so any node connected to the CAN bus can send a message to any other node. Thus, the CAN network usually uses the priority to control accesses to the bus. The CAN network is isolated to the external network, but the attackers hack the digital devices in the vehicle to attack the CAN and ECUs indirectly, which is very dangerous to the vehicle and the public.

There are many attack surfaces of the CAN bus. First is the onboard diagnostics (OBD)-II port, which is used for vehicle status diagnostics, ECU firmware update, and even vehicle control. The OBD-II port is connected to the CAN bus, so the attackers use the OBD-II device and diagnostic software to sniff the message on the bus or control the vehicle [111], [112]. The attackers easily  gain  access  to the CAN bus through OBD-II ports. Second is the media player (e.g., CD player) in the vehicle. The media player

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1711

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

Table 3 Security Threats and Defense Technologies of CAVs

needs to receive the control message from the driver and send the status to the screen (UI), so the media player usually has a connection to the CAN. The attackers easily flash the malicious code to a CD; when the driver plays the CD, the malicious code  attacks the  CAN bus  [113].  In addition,  the attackers utilize  the Bluetooth interface  in the vehicle. Modern vehicles support Bluetooth connec- tions to smartphones. The attackers use smartphones to upload malicious applications via Bluetooth to take over the CAN bus, or they sniff the vehicle status via Bluetooth. It is important to note that the attackers use this interface to attack the vehicle remotely.

Once the attacker hijacks the CAN bus, there are secu- rity threats to the CAN network [111]. First is broadcast trouble. The CAN message is broadcast to all nodes, and the attackers capture and reverse-engineer these messages and inject new messages to induce various actions. Second is a denial-of-service (DoS) attack. The CAN protocol is extremely vulnerable to the DoS attack because of the lim- ited bandwidth. In addition to message flooding attacks,  if one hijacked node keeps claiming highest priority  in  the network,  it will cause all other CAN nodes to back  off, and the whole CAN network will crash. Last is no authentication fields. The CAN message does not contain the authentication fields, which means that any node sends a packet to any other node without an authentication process so the attackers use this to control any node  in  the CAN network.

# V2X Security

With   V2X,   vehicles   access   the   Internet   to   obtain real-time  traffic  data  (e.g.,  real-time  map  and  weather data)  or  leverage  the  cloud  computing  for  autonomous driving [23], and the vehicle also communicates with other nodes in the V2X network via some emerging technologies (e.g.,  DSRC  and  LET-V)  [114].  This  V2X  network  creates many  new  application  scenarios  for  the  CAV,  but  it  also brings more security problems to the vehicles [115]–[117]. The  traditional  Internet  and  ad   hoc   networks  suffer from   many   security   threats,  which   may   occur   in   the

V2X network but with different manifestations. DoS and distributed DoS (DDoS) attacks are two basic attack methods on the Internet. In V2X networks, every node is an attacker or a victim, causing various traffic problems [118]. If the infrastructure is the victim, it cannot provide real-time  service  for  the  nearby   vehicles.  In   contrast, if the vehicle is the victim, it cannot receive the messages from the infrastructure or cloud,  and  the  DoS  attack  also interferes with the performance of other tasks on the vehicle, causing unacceptable latency of some autonomous driving applications [102].

In V2X networks, the attackers create multiple vehicles on the road with the same identity or remain anonymous, which we call the Sybil attack [119].  The  Sybil  attack  may enforce the vehicles running on the road to make  way for the fake vehicles and prevent other vehicles from driving on this road because they are deceived to think there is a traffic jam. Information forgery is also a common attack; a vehicle changes its identity or sends fabricated messages to V2X networks, thus preventing itself from being detected or to shirk their responsibilities [120]. There are many other  traditional  network  threats, such as a replay attack and a block hole attack, but the attack method is similar to the threats mentioned above.

The V2X network brings new types of network nodes, such as the infrastructure and pedestrian so it will have new threats that are rare on the traditional Internet. The first one is about privacy. The communication between V2P and V2I may be based on some short-range protocol (Bluetooth low energy (BLE) and DSRC); if access authen- tication is not strict, privacy of drivers and pedestrians will be exposed [121]. The second one is about infrastructure. If infrastructure (RSU) has been attacked and fake traffic information is broadcast, it influences the running state of the nearby vehicle.

# Security for Edge Network and Platforms

Security is a critical topic for edge computing, so sev- eral works on security for the edge computing system in some general scenarios may provide solutions for security

1712 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

problems in CAV scenarios. The related work is divided into two categories: network in edge and running environment for edge computing.

Bhardwaj   et   al.   [122]   proposed   ShadowNet,   which deploys   the   edge   functions   on   the   distributed   edge infrastructure  and  aggregates  that  information  about  the IoT  traffic  to  detect  an  imminent  IoT-DDoS.  ShadowNet detects IoT-DDoS ten times faster than existing approaches and  also  prevents  82%  of  traffic  to  enter  the  Internet infrastructure,  reducing  security  threats.  Yi  et  al.  [123] summarized  the  method  that  uses  the  software-defined network (SDN)  to solve edge network security  problems, such  as  network  monitoring  and  intrusion  detection  and network  resource  access  control.  This  kind  of  work  will help us solve the related network threats in CAV scenarios.

Ning  et  al.  [124]  evaluated  several  trusted  execution environments  (TEEs)  on  heterogeneous  edge  platforms, such  as  Intel  SGX,  ARM  TrustZone,  and  AMD  SEV,  and deployed  the  TEEs  on  edge  computing  platform  to  effi- ciently improve the security with a low-performance over- head  [124].  Kernel  level  resource  auditing  tool  (KLRA)

[125] is a KLRA for IoT and edge operating system security. KLRA takes fine-grained events measured with low cost and reports the relevant security warning the first time when the behavior of the system is abnormal with this device. This kind of work will help us solve the security problems in the operating system on CAVs.

## CON C L U S I ON

Safety is the most important requirement for autonomous vehicles; hence, the challenge of designing an edge com- puting ecosystem for autonomous vehicles is to deliver enough computing power, redundancy, and security to guarantee the safety of autonomous vehicles. In this paper, we reviewed recent advancements and presented chal- lenges in building edge computing systems for autonomous vehicles, mainly in three categories: edge computing sys- tem designs, V2X applications, and autonomous vehicle security.

In edge computing system designs, we face a system- atic challenge for delivering more computing power with reasonable energy consumption, to guarantee the safety of autonomous vehicles, even at high speed. First, as more autonomous driving edge computing workloads emerge, we need to continue  developing  standard  benchmarks  to more accurately evaluate edge computing systems. Second, although we have seen some recent progress

in utilizing heterogeneous computing architectures for autonomous driving workloads, more research is required to improve the edge computer architecture for autonomous driving. The aim is to deliver more computing power with a limited chip area and energy budget. Third, with the evolu- tion of computer architectures, we need to design runtime layers for efficient mapping of the incoming dynamic work- loads. Fourth, a reliable, robust, and production-quality middleware is still missing to bind various autonomous driving services together. For large-scale deployment of autonomous vehicles, we hope to see more production- quality full-edge computing systems that incorporate inno- vations in these layers.

V2X enables information sharing between vehicles and computation offloading to RSUs. Hence, V2X not only alleviates the stress on edge computing systems but also provides redundancy for sensing, perception, and decision tasks. Therefore, we see V2X as an effective way to improve the safety of autonomous vehicles. Yet, several technical challenges remain such as how the edge computing sys- tems on different vehicles cooperate to make sensing and planning decisions, and how to share the information from infrastructure sensors to autonomous vehicles in real time, as well as to dynamically trade off the cost on infrastruc- ture sensors and on-vehicle sensors. With more research exploring V2X technologies, we hope to see more industrial standards emerging to define how vehicles cooperate with each other and with the infrastructure.

The safety of autonomous vehicles is at risk if security  is compromised at any level. As each autonomous vehicle is equipped with numerous sensors and computing units, an attacker targets one of the sensors, the computing sys- tems, the control systems, or the communication networks to confuse, blind, or even take over control of the vehicle under attack, leading to catastrophic accidents.  Hence,  the challenge is to guarantee security at all levels of the stack. Although in recent years there have been some interesting proposals regarding protecting the security of autonomous vehicles, such as sensor fusion, more research is required before we deploy autonomous vehicles on a large scale. We hope to see more research on defining attack surfaces against autonomous driving edge com- puting ecosystems and proposals to protect autonomous vehicles from these attacks. Most importantly, we urgently need industrial security verification standards to certify whether an autonomous vehicle is secure enough to run on public roads.

RE F E R E NC E S

- S. Liu, J. Tang, Z. Zhang, and J.-L. Gaudiot,

“Computer architectures for autonomous driving,”

Computer, vol. 50, no. 8, pp. 18–25, 2017.

- S. Liu, J. Peng, and J.-L. Gaudiot, “Computer, drive my car!” Computer, vol. 50, no. 1, p. 8, 2017.

- S. Liu, L. Li, J. Tang, S. Wu, and J.-L. Gaudiot, “Creating autonomous vehicle systems,” Synth. Lect. Comput. Sci., vol. 6, no. 1, p. 186, 2017.

- S. Huang and G. Dissanayake, “Convergence and consistency analysis for extended Kalman filter

based SLAM,” IEEE Trans. Robot., vol. 23, no. 5,

pp. 1036–1049, Oct. 2007.

- H. Durrant-Whyte and T. Bailey, “Simultaneous localization and mapping: Part I,” IEEE Robot. Autom. Mag., vol. 13, no. 2, pp. 99–110,

Jun. 2006.

- J. Levinson, M. Montemerlo, and S. Thrun,

“Map-based precision vehicle localization in urban environments,” in Robotics: Science and Systems,

vol. 4. 2007, p. 1.

- A. Krizhevsky, I. Sutskever, and G. E. Hinton,

“ImageNet classification with deep convolutional

neural networks,” in Proc. Adv. Neural Inf. Process. Syst., 2012, pp. 1097–1105.

- L. Bertinetto, J. Valmadre, J. F. Henriques,

A. Vedaldi, and P. H. Torr, “Fully-convolutional siamese networks for object tracking,” in Proc. Eur. Conf. Comput. Vis. Springer, 2016, pp. 850–865.

- D. Ferguson, M. Darms, C. Urmson, and S. Kolski, “Detection, prediction, and avoidance of dynamic obstacles in urban environments,” in Proc. IEEE

Intell. Vehicles Symp., Jun. 2008, pp. 1149–1154.

- S. M. LaValle, Planning Algorithms. Cambridge,

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1713

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

U.K.: Cambridge Univ. Press, 2006.

- J. V. Frasch et al., “An auto-generated nonlinear MPC algorithm for real-time obstacle avoidance of ground vehicles,” in Proc. Eur. Control Conf. (ECC), Jul. 2013, pp. 4136–4141.

- Nvidia Drive PX. Accessed: Dec. 28, 2018.

[Online]. Available: https://www.nvidia.com/en- au/self-driving-cars/drive-px/

- Texas Instruments TDA. Accessed: Dec. 28, 2018. [Online]. Available: http://www.ti.com/

processors/automotive-processors/tdax-adas- socs/overview.html

- CEVA-XM4. Accessed: Dec. 28, 2018. [Online]. Available: https://www.ceva-dsp.com/ product/ceva-xm4/

- Intel Cyclone. Accessed: Dec. 28, 2018. [Online]. Available: https://www.intel.com/content/ www/us/en/fpga/devices.html

- Xilinx Zynq. Accessed: Dec. 28, 2018. [Online].

Available: https://www.xilinx.com/products/ silicon-devices/soc/zynq-7000.html

- Mobileye Eyeq. Accessed: Dec. 28, 2018. [Online]. Available: https://www.mobileye.com/our- technology/evolution-eyeq-chip/

- E. D. Jensen, C. D. Locke, and H. Tokuda,

“A time-driven scheduling model for real-time

operating systems,” in Proc. RTSS, vol. 85, 1985, pp. 112–122.

- D. Hildebrand, “An architectural overview of QNX,” in Proc. USENIX Workshop Microkernels Other Kernel Archit., 1992, pp. 113–126.

- Vxworks. Accessed: Dec. 28, 2018. [Online].

Available: https://www.windriver.com/ products/vxworks/

- M. Quigley et al., “ROS: An open-source Robot Operating System,” in Proc. ICRA Workshop Open Sour. Softw., Kobe, Japan, 2009, vol. 3. no. 2, p. 5.

- ISO 26262. Accessed: Dec. 28, 2018. [Online]. Available: https://www.iso.org/standard/

43464.html

- S. Liu, J. Tang, C. Wang, Q. Wang, and

J.-L. Gaudiot, “A unified cloud platform for autonomous driving,” Computer, vol. 50, no. 12, pp. 42–49, 2017.

- J. Tang, S. Liu, C. Wang, and C. Liu, “Distributed simulation platform for autonomous driving,” in Proc. Int. Conf. Internet Vehicles Springer,  2017, pp. 190–200.

- J. Jiao, “Machine learning assisted high-definition map creation,” in Proc. IEEE 42nd Annu. Comput. Softw. Appl. Conf. (COMPSAC), Jul. 2018,

pp. 367–373.

- A. Geiger, P. Lenz, and R. Urtasun, “Are we ready for autonomous driving? The KITTI vision

benchmark suite,” in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. (CVPR), Jun. 2012,

pp. 3354–3361.

- A. Geiger, P. Lenz, C. Stiller, and R. Urtasun,

“Vision meets robotics: The KITTI dataset,” Int. J. Robot. Res., vol. 32, no. 11, pp. 1231–1237, 2013.

- J. Sturm, N. Engelhard, F. Endres, W. Burgard, and

D. Cremers, “A benchmark for the evaluation of RGB-D SLAM systems,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst. (IROS), Oct. 2012,

pp. 573–580.

- Y. Xiang, R. Mottaghi, and S. Savarese, “Beyond PASCAL: A benchmark for 3D object detection in

the wild,” in Proc. IEEE Winter Conf. Appl. Comput. Vis. (WACV), Mar. 2014, pp. 75–82.

- L. Leal-Taixé, A. Milan, I. Reid, S. Roth, and

K. Schindler. (2015). “MOTChallenge 2015: Towards a benchmark for multi-target tracking.” [Online]. Available: https://arxiv.org/abs/

1504.01942

- A. Milan, L. Leal-Taixé, I. Reid, S. Roth, and

K. Schindler. (2016). “MOT16: A benchmark for multi-object tracking.” [Online]. Available:

https://arxiv.org/abs/1603.00831

- S. K. Venkata et al., “SD-VBS: The San Diego

vision benchmark suite,” in Proc. IEEE Int. Symp. Workload Characterization (IISWC), Oct. 2009, pp. 55–64.

- J. Clemons, H. Zhu, S. Savarese, and T. Austin, “MEVBench: A mobile computer vision benchmarking suite,” in Proc. IEEE Int. Symp.

Workload Characterization (IISWC), Nov. 2011, pp. 91–102.

- L. Nardi et al., “Introducing SLAMBench, a performance and accuracy benchmarking

methodology for SLAM,” in Proc. IEEE Int. Conf. Robot. Autom. (ICRA), May 2015, pp. 5783–5790.

- R. A. Newcombe et al., “KinectFusion: Real-time

dense surface mapping and tracking,” in Proc. 10th IEEE Int. Symp. Mixed Augmented

Reality (ISMAR), Oct. 2011, pp. 127–136.

- Y. Wang, S. Liu, X. Wu, and W. Shi, “CAVBench:

A benchmark suite for connected and autonomous vehicles,” in Proc. IEEE/ACM Symp. Edge Comput. (SEC), Oct. 2018, pp. 30–42.

- S.-C. Lin et al., “The architectural implications of

autonomous driving: Constraints and acceleration,” in Proc. 23rd Int. Conf. Architectural Support Program. Lang. Oper. Syst., 2018,

pp. 751–766.

- J. Tang, B. Yu, S. Liu, Z. Zhang, W. Fang, and

Y. Zhang, “π-SOC heterogeneous SOC

architecture for visual inertial SLAM applications,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst.

(IROS), Oct. 2018, pp. 1–6.

- Z. Zhang, A. A. Suleiman, L. Carlone, V. Sze, and

S. Karaman, “Visual-inertial odometry on chip: An algorithm-and-hardware co-design approach,” Tech. Rep., 2017.

- W. Fang, Y. Zhang, B. Yu, and S. Liu,

“DragonFly+: FPGA-based quad-camera visual SLAM system for autonomous vehicles,” in Proc. IEEE HotChips, 2018, p. 1.

- S. Liu et al., “Cambricon: An instruction set

architecture for neural networks,” ACM SIGARCH Comput. Archit. News, vol. 44, no. 3, pp. 393–405,

2016.

- Y.-H. Chen, J. Emer, and V. Sze, “Eyeriss: A spatial architecture for energy-efficient dataflow for convolutional neural networks,” ACM SIGARCH Comput. Archit. News, vol. 44, no. 3, pp. 367–379, 2016.

- P. Chi et al., “PRIME: A novel

processing-in-memory architecture for neural network computation in ReRAM-based main

memory,” ACM SIGARCH Comput. Archit. News, vol. 44, no. 3, pp. 27–39, Jun. 2016.

- C. Wang, L. Gong, Q. Yu, X. Li, Y. Xie, and

X. Zhou, “DLAU: A scalable deep learning accelerator unit on FPGA,” IEEE Trans.

Comput.-Aided Design Integr. Circuits Syst., vol. 36, no. 3, pp. 513–517, Mar. 2017.

- O. Rahnama, D. Frost, O. Miksik, and P. H. Torr,

“Real-time dense stereo matching with ELAS on FPGA-accelerated embedded devices,” IEEE Robot. Autom. Lett., vol. 3, no. 3, pp. 2008–2015,

Jul. 2018.

- W. Fang, Y. Zhang, B. Yu, and S. Liu, “FPGA-based ORB feature extraction for real-time visual

SLAM,” in Proc. Int. Conf. Field Program. Technol. (ICFPT), Dec. 2017, pp. 275–278.

- X. Wei et al., “Automated systolic array

architecture synthesis for high throughput CNN inference on FPGAs,” in Proc. 54th Annu. Design Autom. Conf., 2017, p. 29.

- Y. Guan et al., “FP-DNN: An automated framework

for mapping deep neural networks onto FPGAs with RTL-HLS hybrid templates,” in Proc. IEEE 25th Annu. Int. Symp. Field-Program. Custom

Comput. Mach. (FCCM), Apr. 2017, pp. 152–159.

- S. Liu, R. N. Pittman, A. Forin, and J.-L. Gaudiot, “Achieving energy efficiency through runtime partial reconfiguration on reconfigurable

systems,” ACM Trans. Embedded Comput. Syst., vol. 12, no. 3, p. 72, 2013.

- G. Hegde et al., “CaffePresso: An optimized library

for deep learning on embedded accelerator-based platforms,” in Proc. CASES, 2016, p. 14.

- M. Malik et al., “Architecture exploration for

energy-efficient embedded vision applications: From general purpose processor to domain specific accelerator,” in Proc. ISVLSI, Jul. 2016, pp. 559–564.

- L. Cavigelli, M. Magno, and L. Benini,

“Accelerating real-time embedded scene labeling with convolutional networks,” in Proc. DAC, 2015, p. 108.

- J. Qiu et al., “Going deeper with embedded FPGA platform for convolutional neural network,” in

Proc. FPGA, 2016, pp. 26–35.

- D. Honegger et al., “Real-time and low latency embedded computer vision hardware based on a combination of FPGA and mobile CPU,” in Proc. IROS, Sep. 2014, pp. 4930–4935.

- M. T. Satria et al., “Real-time system-level

implementation of a telepresence robot using an embedded GPU platform,” in Proc. DATE,

Mar. 2016, pp. 1445–1448.

- A. Vasilyev, N. Bhagdikar, A. Pedram,

S. Richardson, S. Kvatinsky, and M. Horowitz, “Evaluating programmable architectures for

imaging and vision applications,” in Proc. MICRO, 2016, pp. 38–49.

- L. Nardi et al., “Introducing SLAMBench, a performance and accuracy benchmarking

methodology for SLAM,” in Proc. ICRA, May 2015, pp. 5783–5790.

- D. Koufaty, D. Reddy, and S. Hahn, “Bias scheduling in heterogeneous multi-core architectures,” in Proc. 5th Eur. Conf. Comput. Syst., 2010, pp. 125–138.

- J. C. Saez, M. Prieto, A. Fedorova, and

S. Blagodurov, “A comprehensive scheduler for asymmetric multicore systems,” in Proc. 5th Eur. Conf. Comput. Syst., 2010, pp. 139–152.

- V. J. Jiménez, L. Vilanova, I. Gelado, M. Gil,

G. Fursin, and N. Navarro, “Predictive runtime code scheduling for heterogeneous architectures,” in Proc. HiPEAC, vol. 9, 2009, pp. 19–33.

- C.-K. Luk, S. Hong, and H. Kim, “Qilin: Exploiting parallelism on heterogeneous multiprocessors with adaptive mapping,” in Proc. 42nd Annu. IEEE/ACM Int. Symp. Microarchitecture, 2009,

pp. 45–55.

- L. Liu, S. Liu, Z. Zhang, B. Yu, J. Tang, and Y. Xie. (2018). “Pirt: A runtime framework to enable energy-efficient real-time robotic applications on heterogeneous architectures.” [Online]. Available: https://arxiv.org/abs/1802.08359

- H. Utz, S. Sablatnog, S. Enderle, and

G. Kraetzschmar, “Miro-middleware for mobile robot applications,” IEEE Trans. Robot. Autom., vol. 18, no. 4, pp. 493–497, Aug. 2002.

- A. Brooks, T. Kaupp, A. Makarenko, S. Williams, and A. Oreback, “Towards component-based

robotics,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst. (IROS), Aug. 2005, pp. 163–168.

- J.-C. Baillie, “Urbi: Towards a universal robotic low-level programming language,” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst. (IROS), Aug. 2005, pp. 820–825.

- N. Ando, T. Suehiro, K. Kitagaki, T. Kotoku, and W.-K. Yoon, “RT-middleware: Distributed

component middleware for RT (robot

technology),” in Proc. IEEE/RSJ Int. Conf. Intell. Robots Syst. (IROS), Aug. 2005, pp. 3933–3938.

- D. Calisi, A. Censi, L. Iocchi, and D. Nardi,

“OpenRDK: A modular framework for robotic software development,” in Proc. IROS, Sep. 2008, pp. 1872–1877.

- J. Tang, S. Liu, B. Yu, and W. Shi. (2018).

“PI-Edge: A low-power edge computing system for real-time autonomous driving services.” [Online].

Available: https://arxiv.org/abs/1901.04978

- M. Sustrik. (2016). Nanomsg. [Online]. Available: https://www.nanomsg.org

- X. Zhang, Y. Wang, S. Lu, L. Liu, L. Xu, and W. Shi, “OpenEI: An open framework for edge intelligence,” in Proc. IEEE 39th Int. Conf. Distrib. Comput. Syst. (ICDCS), Jul. 2019.

- Q. Zhang et al., “OpenVDAP: An open vehicular data analytics platform for CAVs,” in Proc. IEEE 38th Int. Conf. Distrib. Comput.  Syst.  (ICDCS), Jul. 2018, pp. 1310–1320.

- L. Liu, X. Zhang, Q. Zhang, A. Weinert, Y. Wang, and W. Shi, “AutoVAPS: An IoT-enabled public

safety service on vehicles,” in Proc. 4th Workshop Int. Sci. Smart City Oper. Platforms Eng. (SCOPE), New York, NY, USA, 2019, pp. 41–47.

- H. Stübing et al., “SIM TD: A car-to-x system architecture for field operational tests [topics in automotive networking],” IEEE Commun. Mag.,

1714 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019

Liu et al.: Edge Computing for Autonomous Driving

vol. 48, no. 5, pp. 148–154, May 2010.

- (2010). Deufrako. [Online]. Available: http://deufrako.org/web/index.php

- M. Chevreuil, “IVHW: An inter-vehicle hazard warning system concept within the DEUFRAKO program,” in Proc. e-Saf. Congr. Exhib., Lyon,

France, 2002.

- W. Franz, H. Hartenstein, and M. Mauve,

“Inter-vehicle-communications based on ad hoc networking principles. The FleetNet project,” Universitätsverlag Karlsruhe Karlsruhe, Karlsruhe, Germany, Tech. Rep., 2005.

- H. Hartenstein, B. Bochow, A. Ebner, M. Lott,

M. Radimirsch, and D. Vollmer, “Position-aware ad hoc wireless networks for inter-vehicle communications: The Fleetnet project,” in Proc.

2nd ACM Int. Symp. Mobile Ad Hoc Netw. Comput., 2001, pp. 259–262.

- A. Festag, H. Fußler, H. Hartenstein, A. Sarma, and R. Schmitz, “FleetNet: Bringing car-to-car communication into the real world,” Computer, vol. 4, no. L15, p. 16, 2004.

- D. Reichardt, M. Miglietta, L. Moretti, P. Morsink, and W. Schulz, “CarTALK 2000: Safe and

comfortable driving based upon

inter-vehicle-communication,” in Proc. IEEE Intell. Vehicle Symp., vol. 2, Jun. 2002, pp. 545–550.

- P. L. J. Morsink et al., “CarTALK 2000: Development of a co-operative ADAS based on vehicle-to-vehicle communication,” in Proc. 10th World Congr. Exhib. Intell. Transp. Syst. Services, Madrid, Spain, Nov. 2003.

- Y. Chen, G. Gehlen, G. Jodlauk, C. Sommer, and

- Görg, “A flexible application layer protocol for automotive communications in cellular networks,” in Proc. 15th World Congr. Intell. Transp. Syst.

(ITS), New York City, NY, USA, 2008, pp. 1–9.

- M. Schulze, G. Nocker, and K. Bohm, “PReVENT: A European program to improve active safety,” in Proc. 5th Int. Conf. Intell. Transp. Syst.

Telecommun., Paris, France, 2005.

- A. Hiller, A. Hinsberger, M. Strassberger, and

- Verburg, “Results from the WILLWARN project,” in Proc. 6th Eur. Congr. Exhib. Intell. Transp. Syst. Services, 2007, pp. 1–8.

- A. Festag et al., “‘NoW–network on wheels’:

Project objectives, technology and achievements,” Tech. Rep., 2008.

- G. Toulminet, J. Boussuge, and C. Laurgeau,

“Comparative synthesis of the 3 main European projects dealing with Cooperative Systems (CVIS, SAFESPOT and COOPERS) and description of COOPERS Demonstration Site 4,” in Proc. 11th

IEEE Int. Conf. Intell. Transp. Syst. (ITSC), Oct. 2008, pp. 809–814.

- F. Bonnefoi, F. Bellotti, T. Scendzielorz, and

F. Visintainer, “SAFESPOT applications for infrasructurebased co-operative road safety,” in Proc. 14th World Congr. Exhib. Intell. Transp. Syst. Services, 2007, pp. 1–8.

- A. Papathanassiou and A. Khoryaev, “Cellular V2X as the essential enabler of superior global connected transportation services,” IEEE 5G Tech Focus, vol. 1, no. 2, pp. 1–2, Jun. 2017.

- L. Hobert, A. Festag, I. Llatser, L. Altomare,

F. Visintainer, and A. Kovacs, “Enhancements of V2X communication in support of cooperative autonomous driving,” IEEE Commun. Mag.,

vol. 53, no. 12, pp. 64–70, Dec. 2015.

- A. De La Fortelle et al., “Network of automated vehicles: The AutoNet 2030 vision,” in Proc. ITS World Congr., 2014, pp. 1–10.

- A. Marjovi, M. Vasic, J. Lemaitre, and A. Martinoli, “Distributed graph-based convoy control for networked intelligent vehicles,” in Proc. IEEE

Intell. Vehicles Symp. (IV), Jun. 2015, pp. 138–143.

- U. Khan, P. Basaras, L. Schmidt-Thieme,

A. Nanopoulos, and D. Katsaros, “Analyzing cooperative lane change models for connected vehicles,” in Proc. Int. Conf. Connected Vehicles Expo (ICCVE), Nov. 2014, pp. 565–570.

- A. Kesting, M. Treiber, and D. Helbing, “General lane-changing model MOBIL for car-following models,” Transp. Res. Rec., vol. 1999,

pp. 86–94, Jan. 2007.

- (2018). Miovision Unveils the World’s Smartest Intersection in Detroit. [Online]. Available:

https://miovision.com/press/miovision-unveils- the-worlds-smartest-intersection-in-detroit/

- K. Zheng, Q. Zheng, P. Chatzimisios, W. Xiang, and

Y. Zhou, “Heterogeneous vehicular networking:

A survey on architecture, challenges, and

solutions,” IEEE Commun. Surveys Tuts., vol. 17, no. 4, pp. 2377–2396, 4th Quart., 2015.

- J. B. Kenney, “Dedicated short-range communications (DSRC) standards in the United

States,” Proc. IEEE, vol. 99, no. 7, pp. 1162–1182,

Jul. 2011.

- K. Zheng, Q. Zheng, H. Yang, L. Zhao, L. Hou, and

P. Chatzimisios, “Reliable and efficient autonomous driving: The need for heterogeneous vehicular networks,” IEEE Commun. Mag., vol. 53, no. 12, pp. 72–79, Dec. 2015.

- J. N. G. Isento, J. J. P. C. Rodrigues, J. A. F. F. Dias,

M. C. G. Paula, and A. Vinel, “Vehicular

delay-tolerant networks? A novel solution for vehicular communications,” IEEE Intell. Transp. Syst. Mag., vol. 5, no. 4, pp. 10–19, Oct. 2013.

- J. A. Dias, J. J. Rodrigues, N. Kumar, and

K. Saleem, “Cooperation strategies for vehicular delay-tolerant networks,” IEEE Commun. Mag., vol. 53, no. 12, pp. 88–94, Dec. 2015.

- A. Ghosh, V. V. Paranthaman, G. Mapp,

O. Gemikonakli, and J. Loo, “Enabling seamless V2I communications: Toward developing

cooperative automotive applications in VANET systems,” IEEE Commun. Mag., vol. 53, no. 12, pp. 80–86, Dec. 2015.

- (2018). Automotive Edge Computing Consortium. [Online]. Available: https://aecc.org/

- J. G. Andrews et al., “What will 5G be?” IEEE J. Sel. Areas Commun., vol. 32, no. 6,

pp. 1065–1082, Jun. 2014.

- J. Petit and S. E. Shladover, “Potential cyberattacks on automated vehicles,” IEEE Trans. Intell. Transp. Syst., vol. 16, no. 2, pp. 546–556, Apr. 2015.

- J. Petit, B. Stottelaar, M. Feiri, and F. Kargl, “Remote attacks on automated vehicles sensors: Experiments on camera and lidar,” Black Hat Europe, vol. 11, 2015.

- K. N. Truong, S. N. Patel, J. W. Summet, and

G. D. Abowd, “Preventing camera recording by designing a capture-resistant environment,” in

Proc. Int. Conf. Ubiquitous Comput. Springer, 2005, pp. 73–86.

- R. T. Ioannides, T. Pany, and G. Gibbons, “Known vulnerabilities of global navigation satellite

systems, status, and potential mitigation techniques,” Proc. IEEE, vol. 104, no. 6, pp. 1174–1194, Jun. 2016.

- C. Yan, W. Xu, and J. Liu, “Can you trust

autonomous vehicles: Contactless attacks against sensors of self-driving vehicle,” DEF CON, vol. 24, 2016.

- S.-Y. Jeong et al., “A study on ROS vulnerabilities and countermeasure,” in Proc. Companion

ACM/IEEE Int. Conf. Hum.-Robot Interact., 2017, pp. 147–148.

- F. J. R. Lera, J. Balsa, F. Casado, C. Fernández,

F. M. Rico, and V. Matellán, “Cybersecurity in autonomous systems: Evaluating the performance of hardening ROS,” Málaga, Spain, Tech. Rep.,

vol. 47, 2016.

- J. McClean, C. Stull, C. Farrar, and D. Mascareñas, “A preliminary cyber-physical security assessment of the robot operating system (ROS),” Proc. SPIE, vol. 8741, May 2013, Art. no. 874110.

- K. H. Johansson, M. Törngren, and L. Nielsen,

“Vehicle applications of controller area network,” in Handbook of Networked and Embedded Control Systems Springer, 2005, pp. 741–765.

- K. Koscher et al., “Experimental security analysis of a modern automobile,” in Proc. IEEE Symp.

Secur. Privacy (SP), May 2010, pp. 447–462.

- Q. Wang and S. Sawhney, “VeCure: A practical security framework to protect the CAN bus of

vehicles,” in Proc. Int. Conf. Internet Things (IOT), Oct. 2014, pp. 13–18.

- S. Checkoway et al., “Comprehensive experimental analyses of automotive attack surfaces,” in Proc. USENIX Secur. Symp.,

San Francisco, CA, SA, 2011, pp. 77–92.

- J. Hu et al., “Link level performance comparison between LTE V2X and DSRC,” J. Commun. Inf. Netw., vol. 2, no. 2, pp. 101–112, 2017.

- M. Raya and J.-P. Hubaux, “Securing vehicular ad hoc networks,” J. Comput. Secur., vol. 15, no. 1, pp. 39–68, 2007.

- R. G. Engoulou, M. Bellaíche, S. Pierre, and

A. Quintero, “VANET security surveys,” Comput. Commun., vol. 44, pp. 1–13,

May 2014.

- Y. Yang, Z. Wei, Y. Zhang, H. Lu, K.-K. R. Choo, and H. Cai, “V2X security: A case study of

anonymous authentication,” Pervas. Mobile Comput., vol. 41, pp. 259–269, Oct. 2017.

- A. M. Malla and R. K. Sahu, “Security attacks with an effective solution for dos attacks in VANET,”

Int. J. Comput. Appl., vol. 66, no. 22, pp. 1–5, 2013.

- B. Yu, C.-Z. Xu, and B. Xiao, “Detecting sybil

attacks in VANETs,” J. Parallel Distrib. Comput., vol. 73, no. 6, pp. 746–756, 2013.

- J. Petit, M. Feiri, and F. Kargl, “Spoofed data detection in VANETs using dynamic thresholds,” in Proc. VNC, 2011, pp. 25–32.

- J. Liu and J. Liu, “Intelligent and connected vehicles: Current situation, future directions, and challenges,” IEEE Commun. Standards Mag.,

vol. 2, no. 3, pp. 59–65, Sep. 2018.

- K. Bhardwaj, J. C. Miranda, and A. Gavrilovska, “Towards IoT-DDoS prevention using edge

computing,” in Proc. USENIX Workshop Hot Topics Edge Comput. (HotEdge 18), 2018.

- S. Yi, Z. Qin, and Q. Li, “Security and privacy

issues of fog computing: A survey,” in Proc. Int. Conf. Wireless Algorithms, Syst., Appl. Springer, 2015, pp. 685–695.

- Z. Ning, J. Liao, F. Zhang, and W. Shi, “Preliminary study of trusted execution environments on

heterogeneous edge platforms,” in Proc. IEEE/ACM Symp. Edge Comput. (SEC), Oct. 2018, pp. 421–426.

- D. Li, Z. Zhang, W. Liao, and Z. Xu, “KLRA:

A kernel level resource auditing tool for IoT operating system security,” in Proc. IEEE/ACM Symp. Edge Comput. (SEC), Oct. 2018,

pp. 427–432.

- B. Boroujerdian, H. Genc, S. Krishnan, W. Cui, A. Faust, and V. Reddi, “MAVBench: Micro aerial vehicle benchmarking,” in Proc. 51st Annu. IEEE/ACM Int. Symp. Microarchitecture (MICRO), 2018, pp. 894–907.

- Y. Wang, L. Liu, X. Zhang, and W. Shi, “HydraOne: An indoor experimental research and education platform for CAVs,” in Proc. 2nd USENIX Workshop Hot Topics Edge Comput. (HotEdge). Renton, WA, USA: USENIX Association, 2019.

Vol. 107, No. 8, August 2019 | PROCEEDINGS OF THE IEEE 1715

Liu et al.: Edge Computing for Autonomous Driving: Opportunities and Challenges

AB O U T T H E  A U TH OR S

Shaoshan Liu (Senior Member, IEEE) received the Ph.D. degree in computer engineering from the University of California at Irvine, Irvine, CA, USA, in 2010.

He was a Founding Member of Baidu USA, Sunnyvale, CA, USA, and the Baidu Autonomous Driving Unit,  where he was     in charge of system integration of autonomous driving systems. He is currently the Founder and CEO of PerceptIn, Fremont, CA, USA, a company focusing on providing visual perception solutions for autonomous robots and vehicles. He has authored or coau- thored more than 40 high-quality research papers. He holds more than 150 U.S. international patents on robotics and autonomous driving. He is also the Lead Author of the best-selling textbook Creating Autonomous Vehicle Systems, which is the first technical overview of autonomous vehicles written for a general computing and engineering audience. His current research interests include computer architecture, deep learning infrastructure, robotics, and autonomous driving.

Dr. Liu co-founded the  IEEE  Special  Technical  Community on Autonomous Driving Technologies to bridge communications between global autonomous driving researchers and practition- ers and serves as the Founding Vice President. He is an ACM Distinguished Speak and an IEEE Computer Society Distinguished Speaker.

Liangkai Liu received the B.S. degree in telecommunication engi- neering from Xidian University, Xi’an, China, in 2017. He is cur- rently working toward the Ph.D. degree at Wayne State University, Detroit, MI, USA.

His current research interests include edge computing, distrib- uted systems, and autonomous driving.

Jie Tang (Member, IEEE) received the B.E. degree in computer science from the National University of Defense Technology, Chang- sha, China, in 2006, and the Ph.D. degree in computer science from the Beijing Institute of Technology, Beijing, China, in 2012.

She was a Visiting Researcher with the Embedded Systems Center, University of California at Irvine, Irvine, CA, USA, and     a Research Scientist with Intel China Runtime Technology Lab, Beijing. She is currently an Associate Professor with the School of Computer Science and Engineering, South China University of Tech- nology, Guangzhou, China. Her current research interests include research on computer architecture, autonomous driving, cloud, and run-time system.

Dr. Tang is a Founding Member and the Secretary of the IEEE Computer Society Special Technical Community on Autonomous Driving Technologies.

Bo Yu (Member, IEEE) received the B.S. degree in electronic technology and science from Tianjin University,  Tianjin,  China, in 2006, and the Ph.D. degree from the Institute of Microelectronics, Tsinghua University, Beijing, China, in 2012.

He is currently the CTO of PerceptIn, Fremont, CA, USA, a com- pany focusing on providing visual perception solutions for robotics and autonomous driving. His current research interests include algorithm and systems for robotics and autonomous vehicles.

Dr. Yu is a Founding Member of the IEEE Special Technical Community on Autonomous Driving.

Yifan Wang received the B.S. degree in electronic information engineering from Tianjin University, Tianjin, China, in 2014. He is currently working toward the Ph.D. degree at the Institute of Com- puting Technology, Chinese Academy of Sciences, Beijing, China.

His current research interests include computer architecture, edge computing, and connected and autonomous vehicles.

Weisong Shi (Fellow, IEEE) received the B.S. degree in computer engineering from Xidian University, Xi’an, China, in 1995, and the Ph.D. degree in computer engineering from the Chinese Academy of Sciences, Beijing, China, in 2000.

He is currently the Charles H. Gershenson Distinguished Faculty Fellow and a Professor of computer science with Wayne State University, Detroit, MI, USA. His current research interests include edge computing, computer systems, energy efficiency, and wire- less health.

Dr. Shi was a recipient of the National Outstanding Ph.D. Disser- tation Award of China and the NSF CAREER Award.

1716 PROCEEDINGS OF THE IEEE | Vol. 107, No. 8, August 2019